Odour Monitoring - Narrogin

# Project 32-2006

A community odour monitoring program was established in conjunction with the Narrogin community in 2007 in an attempt to establish baseline data on odour being experienced from agricultural sources in the town.

## Background

Continuing community debate over the odour being reportedly being generated from Narrogin Beef Producers lead to the Department of Agriculture and Food to establish a data collection process to attempt to quantify basic environmental data to a degree of objectivity to what was becoming a highly emotive subjective debate.

## Research Background

Initial research sought to identify the most appropriate way in which objective data was collected.  Consultation was sought with The Odour Unit, Department of Environment and Conservation, Department of Primary industries (QLD), Murdoch University, and Feedlot Services Australia.

The first approach was to take samples of the air for analysis.  The analysis was then to be undertaken by the Odour Unit in Perth.  This was attempted and discounted on the basis that it was very costly, it required consistent capture technique and timing (which was difficult to maintain) and involved getting the samples to Perth immediately for analysis.

The second approach was the use of objective measurement.  This involved air samples anaysised to identify likely offending elements.  This was undertaken in August 2006. Hydrogen Sulphide and Ammonia were identified as the most likely offence gasses from the thirty plus that made up the emissions from the feed lot.  Hand held electronic measuring equipment was obtained from the United States to field sample levels of H2S and NH4  (Gray Wolf Toxic Gas Probe  TG-502 calibrated for temperature, TVOC, NH4 and H2S).  Field trials with this equipment identified that the low levels of gases, although easily detected by the human nose were only marginally detectable with the sensors available.  It was subsequently decided to use the equipment for calibration purposes rather than ongoing monitoring.

The third approach was to consider using a network of people that were cross referenced against a standard.  The selection of the participants involved a lengthy process that essentially identified those individuals who were both prepared to sample regularly and to were considered as independent enough not to swayed by bias either for or against Narrogin Beef Producers.  Many of the data collectors raised concerns about being involved as it was perceived that the circumstances surrounding Narrogin Beef Producers was becoming more controversial.  Assurances were given by the Department of Agriculture and Food that under no circumstances would the identity of collectors be made public.  Even so, many of the possible collectors despite being concerned about the issue were not prepared to participate.

Commencing in May 2007, eight different localities were selected with several collectors in each locality (Initially totalling 23 individuals collecting for a three month period).  A review was conducted at the end of the first three months resulting in many of the participants not believing they would continue collecting data for reasons such as personal logistics, inability to remain objective; or just a desire to distance themself from the issue.

As of August 2007 a modified program continued with only three localities, involving 17 different collectors.  This group continued to collect data until May 2009.  The new localities were restricted to the Narrogin town site (east, central and west).  17 different collectors were necessary to ensure at least one sample was taken, three times per day, seven days a week in each locality.

.

WEST

Central

East

Narrogin Townsite localities

Data collection involved sniffing the air at three set times of the day, 9am, 3pm, and 9pm.  The air quality was then assessed on a scale of 0-6 (0 being no odour, 6 being almost to the point of gagging).  This result was then recorded on a log sheet which was then submitted to the Department of Agriculture and Food for compilation.

The Department of Agriculture and food provided a staff member to act as the standard over this period.  This involved checking in on each data collector once a fortnight and independently rating air quality at the same time.  This was seen as the most effective way to normalise the data and maintain consistency

## Observation

There was a very high degree of consistency between the three localities.  The only variation between localities occurred on days shaded in pink.  On all other days there was no significant difference in the reading recorded from the three town localities.

## Results

Note:  The data can not be considered as being exclusively the result of Narrogin Beef as it is possible that there are other odour sources contributing.

| Combined data for the town of Narrogin |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Temp | RH | Cld | Dir | Speed | Odour 9am | Temp | RH | Cld | Dir | Speed | Odour 3pm | Odour 9pm |
| 15/05/2007 | 11.3 | 61 | 8 | SSE | 11 | 1 | 14.8 | 62 | 8 | SSE | 4 | 0 | 0 |
| 16/05/2007 | 10 | 80 | 8 | Calm | 0 | 0 | 13.8 | 61 | 7 | SSE | 2 | 0 | 0 |
| 17/05/2007 | 11.6 | 65 | 0 | N | 4 | 0 | 18.5 | 38 | 1 | NW | 8 | 0 | 0 |
| 18/05/2007 | 10.8 | 78 | 0 | SSW | 2 | 0 | 14.9 | 44 | 5 | S | 4 | 1 | 2 |
| 19/05/2007 | 11.6 | 64 | 3 | Calm | 0 | 0 | 14.8 | 60 | 7 | NW | 18 | 0 | 0 |
| 20/05/2007 | 11.3 | 80 | 6 | SW | 4 | 0 | 15.6 | 43 | 5 | SSW | 4 | 2 | 2 |
| 21/05/2007 | 6.6 | 87 | 1 | Calm | 0 | 0 | 15.7 | 43 | 3 | NW | 2 | 0 | 0 |
| 22/05/2007 | 10.5 | 76 | 2 | Calm | 0 | 0 | 17.2 | 47 | 2 | NW | 4 | 0 | 0 |
| 23/05/2007 | 13 | 88 | 8 | NW | 5 | 0 | 19.4 | 53 | 5 | NW | 26 | 0 | 0 |
| 24/05/2007 | 15 | 87 | 3 | WNW | 8 | 0 | 19.5 | 52 | 3 | WNW | 15 | 0 | 0 |
| 25/05/2007 | 14.4 | 94 | 8 | NW | 4 | 0 | 20 | 53 | 5 | WNW | 11 | 0 | 0 |
| 26/05/2007 | 13 | 94 | 8 | W | 2 | 0 | 15.1 | 53 | 4 | WSW | 9 | 0 | 2 |
| 27/05/2007 | 10.5 | 95 | 7 | Calm | 0 | 0 | 12.9 | 78 | 8 | W | 4 | 0 | 0 |
| 28/05/2007 | 10.2 | 85 | 7 | SSW | 11 | 3 | 13.6 | 46 | 5 | SSW | 15 | 2 | 3 |
| 29/05/2007 | 8.7 | 84 | 8 | Calm | 0 | 0 | 14.3 | 50 | 8 | WNW | 2 | 0 | 0 |
| 30/05/2007 | 10.5 | 76 | 8 | NNW | 2 | 0 | 15.5 | 62 | 8 | Calm | 0 | 0 | 0 |
| 31/05/2007 | 8.8 | 95 | 2 | Calm | 0 | 0 | 17.8 | 41 | 1 | NNE | 2 | 0 | 0 |
| 1/06/2007 | 11.9 | 74 | 1 | Calm | 0 | 0 | 19.5 | 31 | 1 | Calm | 0 | 0 | 0 |
| 2/06/2007 | 10 | 89 | 1 | Calm | 0 | 0 | 18.9 | 47 | 6 | ENE | 2 | 0 | 0 |
| 3/06/2007 | 11.2 | 84 | 8 | SSE | 4 | 0 | 15.7 | 48 | 1 | SSE | 8 | 0 | 0 |
| 4/06/2007 | 7.3 | 99 | 7 | SSE | 2 | 0 | 14 | 51 | 6 | ESE | 9 | 0 | 0 |
| 5/06/2007 | 9.8 | 87 | 7 | E | 9 | 0 | 15.8 | 46 | 5 | E | 11 | 0 | 0 |
| 6/06/2007 | 9.1 | 69 | 4 | NE | 15 | 0 | 16.1 | 36 | 8 | NE | 15 | 0 | 0 |
| 7/06/2007 | 11.7 | 58 | 7 | NNE | 22 | 0 |  |  |  |  |  |  |  |
| 8/06/2007 | 13.8 | 96 | 7 | N | 2 | 0 | 18 | 65 | 4 | WSW | 4 | 1 | 2 |
| 9/06/2007 | 8.6 | 99 | 0 | Calm | 0 | 0 | 15.7 | 50 | 4 | SW | 2 | 0 | 0 |
| 10/06/2007 | 6.4 | 99 | 0 | Calm | 0 | 0 | 15 | 49 | 5 | Calm | 0 | 0 | 0 |
| 11/06/2007 | 6.8 | 90 | 6 | Calm | 0 | 0 | 15.5 | 46 | 3 | Calm | 0 | 0 | 1 |
| 12/06/2007 | 10.4 | 64 | 6 | NNE | 2 | 0 | 20.5 | 33 | 2 | NE | 11 | 0 | 0 |
| 13/06/2007 | 12.8 | 49 | 8 | N | 15 | 0 | 19.2 | 58 | 8 | NNW | 18 | 0 | 0 |
| 14/06/2007 | 12.5 | 94 | 3 | Calm | 0 | 0 | 16 | 54 | 5 | SSW | 2 | 0 | 0 |
| 15/06/2007 | 6.9 | 100 |  | Calm | 0 | 0 | 15.8 | 52 | 5 | SE | 4 | 0 | 0 |
| 16/06/2007 | 9 | 100 | 8 | SSE | 2 | 0 |  |  |  |  |  |  |  |
| 17/06/2007 | 7.4 | 83 | 0 | Calm | 0 | 0 | 14.1 | 40 | 3 | ENE | 15 | 0 | 0 |
| 18/06/2007 | 7.7 | 97 | 8 | SE | 2 | 0 | 14 | 64 | 4 | ESE | 2 | 0 | 0 |
| 19/06/2007 | 8.3 | 96 | 8 | SE | 4 | 0 | 13.6 | 61 | 7 | ESE | 2 | 0 | 0 |
| 20/06/2007 | 7 | 93 | 8 | SE | 2 | 0 | 12.1 | 60 | 7 | ENE | 2 | 0 | 0 |
| 21/06/2007 | 8.3 | 88 | 8 | NE | 2 | 0 | 13.5 | 56 | 5 | NE | 2 | 0 | 0 |
| 22/06/2007 | 9.2 | 48 | 6 | N | 28 | 0 | 14.5 | 35 | 8 | N | 28 | 0 | 0 |
| 23/06/2007 | 10 | 93 | 7 | NNW | 4 | 0 | 10.6 | 80 | 6 | NW | 4 | 0 | 0 |
| 24/06/2007 | 7.6 | 85 | 1 | NW | 2 | 0 | 13.6 | 66 | 6 | NW | 4 | 0 | 0 |
| 25/06/2007 | 9 | 100 | 8 | Calm | 0 | 0 |  |  |  |  |  |  |  |
| 26/06/2007 | 10.2 | 77 | 0 | NNW | 11 | 0 | 17.4 | 37 | 0 | NNW | 15 | 0 | 0 |
| 27/06/2007 | 11.7 | 38 | 6 | NNW | 18 | 0 | 16.1 | 46 | 8 | NNW | 15 | 0 | 0 |
| 28/06/2007 | 13.5 | 94 | 5 | Calm | 0 | 0 | 17 | 61 | 4 | NW | 11 | 0 | 0 |
| 29/06/2007 | 11 | 100 | 7 | NNW | 4 | 1 | 17.8 | 58 | 5 | NNW | 18 | 0 | 0 |
| 30/06/2007 | 12.2 | 91 | 2 | Calm | 0 | 0 | 15.2 | 72 | 6 | WNW | 2 | 0 | 0 |
| 1/07/2007 | 12.5 | 96 | 8 | NNW | 11 | 0 | 14.5 | 86 | 8 | NW | 22 | 0 | 0 |
| 2/07/2007 | 12 | 76 | 2 | NNW | 22 | 0 | 12.8 | 76 | 7 | NW | 37 | 0 | 0 |
| 3/07/2007 | 8.6 | 84 | 2 | NW | 15 | 0 | 13.5 | 62 | 3 | WNW | 18 | 0 | 0 |
| 4/07/2007 | 11.6 | 81 | 7 | WSW | 4 | 1 | 16.2 | 55 | 4 | WNW | 9 | 0 | 0 |
| 5/07/2007 | 12 | 76 | 7 | WNW | 2 | 0 | 15.6 | 68 | 4 | WSW | 2 | 0 | 0 |
| 6/07/2007 | 12 | 88 | 7 | Calm | 0 | 0 | 15.5 | 61 | 8 | N | 8 | 0 | 0 |
| 7/07/2007 | 11 | 70 | 0 | N | 15 | 0 | 18.1 | 43 | 3 | N | 11 | 0 | 0 |
| 8/07/2007 | 12.4 | 77 | 6 | N | 15 | 0 | 15.5 | 80 | 7 | NW | 11 | 0 | 0 |
| 9/07/2007 | 4.5 | 92 | 0 | Calm | 0 | 0 | 13 | 61 | 4 | SSW | 4 | 1 | 1 |
| 10/07/2007 | 7.4 | 83 | 7 | ESE | 2 | 0 | 14 | 57 | 1 | SE | 4 | 0 | 0 |
| 11/07/2007 | 8.4 | 81 | 5 | SE | 2 | 0 | 15.7 | 60 | 4 | SE | 2 | 0 | 0 |
| 12/07/2007 | 10.7 | 86 | 7 | Calm | 0 | 0 | 17.8 | 51 | 6 | Calm | 0 | 0 | 0 |
| 13/07/2007 | 11.3 | 63 | 8 | NNW | 2 | 0 | 16.4 | 41 | 7 | Calm | 0 | 0 | 0 |
| 14/07/2007 | 10.9 | 70 | 7 | Calm | 0 | 0 | 16 | 43 | 2 | Calm | 0 | 0 | 0 |
| 15/07/2007 | 6.7 | 90 | 1 | Calm | 0 | 0 | 17.1 | 43 | 1 | Calm | 0 | 0 | 0 |
| 16/07/2007 | 9.7 | 78 | 7 | WNW | 2 | 0 | 14.6 | 58 | 7 | SW | 2 | 0 | 1 |
| 17/07/2007 | 9 | 96 | 8 | Calm | 0 | 0 | 15.5 | 50 | 4 | NE | 4 | 0 | 0 |
| 18/07/2007 | 10.8 | 64 | 4 | N | 4 | 0 | 19.9 | 36 | 6 | NW | 9 | 0 | 0 |
| 19/07/2007 | 13.2 | 35 | 8 | N | 15 | 0 | 14.1 | 57 | 8 | N | 18 | 0 | 0 |
| 20/07/2007 | 12 | 91 | 7 | NNW | 8 | 0 | 15.5 | 65 | 5 | NW | 4 | 0 | 0 |
| 21/07/2007 | 14 | 93 | 8 | N | 33 | 0 | 12 | 80 | 8 | NW | 18 | 0 | 0 |
| 22/07/2007 | 10.3 | 89 | 4 | NW | 22 | 0 | 12.6 | 84 | 6 | NW | 18 | 0 | 0 |
| 23/07/2007 | 9 | 100 | 8 | NNW | 2 | 0 | 14.5 | 84 | 5 | N | 15 | 0 | 0 |
| 24/07/2007 | 10 | 83 | 2 | NW | 4 | 0 | 13.1 | 68 | 4 | WNW | 15 | 0 | 0 |
| 25/07/2007 | 10.6 | 92 | 8 | NW | 8 | 0 | 12.9 | 77 | 5 | WNW | 4 | 0 | 1 |
| 26/07/2007 | 8.5 | 99 | 4 | Calm | 0 | 0 | 14.9 | 60 | 3 | NNW | 4 | 0 | 0 |
| 27/07/2007 | 11 | 70 | 2 | N | 4 | 0 | 17.9 | 49 | 6 | N | 22 | 0 | 0 |
| 28/07/2007 | 11.9 | 74 | 1 | NW | 9 | 0 | 15.1 | 66 | 5 | WNW | 18 | 0 | 0 |
| 29/07/2007 | 13.9 | 87 | 4 | NW | 15 | 0 | 16.1 | 74 | 6 | NW | 18 | 0 | 0 |
| 30/07/2007 | 13.3 | 91 | 8 | NNW | 28 | 0 | 12.2 | 95 | 8 | NW | 18 | 0 | 0 |
| 31/07/2007 | 10.1 | 80 | 2 | NW | 15 | 0 | 14.6 | 51 | 2 | WNW | 18 | 0 | 0 |
| 1/08/2007 | 7 | 93 | 7 | N | 4 | 0 | 12 | 84 | 8 | NNW | 4 | 0 | 0 |
| 2/08/2007 | 10 | 75 | 0 | WSW | 15 | 0 | 12.9 | 47 | 6 | SW | 7 | 4 | 4 |
| 3/08/2007 | 8.4 | 81 | 1 | SW | 2 | 0 | 12.4 | 61 | 7 | SSW | 2 | 1 | 2 |
| 4/08/2007 | 7.4 | 94 | 1 | Calm | 0 | 0 | 14 | 52 | 4 | NNW | 4 | 0 | 0 |
| 5/08/2007 | 9.9 | 82 | 1 | Calm | 0 | 0 | 14 | 62 | 5 | Calm | 0 | 0 | 0 |
| 6/08/2007 | 9 | 84 | 7 | Calm | 0 | 0 | 16.2 | 47 | 4 | WNW | 2 | 0 | 0 |
| 7/08/2007 | 9.8 | 90 | 8 | S | 2 | 0 | 12.5 | 66 | 8 | SE | 2 | 0 | 0 |
| 8/08/2007 | 7.7 | 84 | 0 | S | 2 | 0 | 14 | 43 | 5 | ESE | 4 | 0 | 0 |
| 9/08/2007 | 7 | 87 | 0 | SE | 2 | 0 | 13.8 | 46 | 4 | ESE | 4 | 1 | 1 |
| 10/08/2007 | 7.9 | 84 | 3 | SE | 2 | 0 | 13 | 55 | 6 | E | 2 | 0 | 0 |
| 11/08/2007 | 3.4 | 93 | 2 | Calm | 0 | 0 | 14.8 | 41 | 4 | SE | 4 | 0 | 0 |
| 12/08/2007 | 5.4 | 100 | 0 | Calm | 0 | 0 | 13.2 | 54 | 5 | ESE | 4 | 0 | 0 |
| 13/08/2007 | 6.6 | 91 | 1 | SE | 2 | 0 | 15.4 | 37 | 4 | ESE | 2 | 0 | 0 |
| 14/08/2007 | 9.5 | 62 | 0 | Calm | 0 | 0 | 16.4 | 31 | 3 | ENE | 2 | 0 | 0 |
| 15/08/2007 | 9.5 | 51 | 1 | NNE | 4 | 0 | 17.6 | 32 | 6 | NNE | 11 | 0 | 0 |
| 16/08/2007 | 11.8 | 43 | 0 | N | 9 | 0 | 18.2 | 34 | 2 | NNE | 4 | 0 | 0 |
| 17/08/2007 | 12 | 43 | 0 | N | 11 | 0 | 19 | 26 | 0 | NNE | 9 | 0 | 0 |
| 18/08/2007 | 11 | 48 | 0 | N | 7 | 0 | 19 | 26 | 2 | NNW | 11 | 0 | 0 |
| 19/08/2007 | 10.5 | 89 | 8 | NW | 4 | 0 | 11.8 | 86 | 7 | Calm | 0 | 0 | 0 |
| 20/08/2007 | 8.9 | 87 | 1 | Calm | 0 | 0 | 12.6 | 59 | 7 | SE | 7 | 0 | 1 |
| 21/08/2007 | 8.5 | 77 | 8 | ESE | 9 | 1 | 11.6 | 53 | 3 | ESE | 9 | 1 | 0 |
| 22/08/2007 | 8.4 | 67 | 1 | ENE | 19 | 0 | 14 | 45 | 4 | NE | 7 | 0 | 0 |
| 23/08/2007 | 9.6 | 60 | 1 | NNE | 11 | 0 | 16.4 | 33 | 0 | NNW | 7 | 0 | 0 |
| 24/08/2007 | 10.9 | 48 | 8 | Calm | 0 | 0 | 18.6 | 34 | 6 | NNW | 4 | 0 | 0 |
| 25/08/2007 | 12.2 | 93 | 7 | SSE | 4 | 1 | 15.1 | 50 | 6 | SSW | 4 | 1 | 0 |
| 26/08/2007 | 9.5 | 69 | 5 | SE | 4 | 0 | 16.3 | 49 | 1 | SE | 4 | 0 | 0 |
| 27/08/2007 | 10.5 | 73 | 4 | NE | 4 | 0 | 18.4 | 42 | 2 | NNE | 4 | 0 | 0 |
| 28/08/2007 | 12 | 88 | 5 | Calm | 0 | 0 | 17.9 | 55 | 6 | SW | 4 | 2 | 2 |
| 29/08/2007 | 12 | 82 | 7 | SSW | 2 | 0 | 12.7 | 74 | 6 | SSW | 7 | 2 | 4 |
| 30/08/2007 | 7.4 | 79 | 5 | SSW | 4 | 1 | 11 | 53 | 8 | SW | 4 | 1 | 3 |
| 31/08/2007 | 9.6 | 73 | 1 | NNW | 9 | 0 | 13 | 74 | 7 | NW | 15 | 0 | 0 |
| 1/09/2007 | 11.5 | 90 | 3 | Calm | 0 | 0 | 16.8 | 48 | 5 | NW | 2 | 0 | 0 |
| 2/09/2007 | 13.3 | 74 | 1 | NNE | 2 | 0 | 19.1 | 43 | 5 | N | 7 | 0 | 0 |
| 3/09/2007 | 15.1 | 62 | 0 | NW | 2 | 0 | 22 | 25 | 5 | N | 11 | 0 | 0 |
| 4/09/2007 | 11.8 | 87 | 8 | S | 4 | 2 | 18.9 | 51 | 5 | SSE | 22 | 0 | 0 |
| 5/09/2007 | 11.7 | 80 | 7 | S | 11 | 1 | 17.6 | 48 | 3 | SE | 7 | 0 | 0 |
| 6/09/2007 | 10.4 | 59 | 1 | SSE | 15 | 0 | 14.2 | 45 | 4 | SSE | 9 | 0 | 1 |
| 7/09/2007 | 10.1 | 68 | 3 | SE | 4 | 1 | 15.5 | 49 | 4 | NNW | 4 | 0 | 0 |
| 8/09/2007 | 8.9 | 85 | 7 | Calm | 0 | 0 | 17.9 | 39 | 7 | NNW | 11 | 0 | 0 |
| 9/09/2007 | 14 | 61 | 0 | Calm | 0 | 0 | 20.5 | 33 | 1 | WNW | 4 | 0 | 0 |
| 10/09/2007 | 13 | 73 | 4 | NNE | 2 | 0 | - | - | - | - | - | 0 | 0 |
| 11/09/2007 | 13.5 | 87 | 6 | WNW | 15 | 0 | 15.8 | 71 | 7 | WNW | 19 | 0 | 0 |
| 12/09/2007 | 14 | 68 | 8 | NW | 28 | 0 | 16.5 | 62 | 6 | NW | 37 | 0 | 0 |
| 13/09/2007 | 10.8 | 66 | 6 | WSW | 1 | 0 | 15.1 | 42 | 6 | WSW | 22 | 1 | 1 |
| 14/09/2007 | 13.2 | 79 | 7 | W | 19 | 0 | 17 | 35 | 5 | SW | 22 | 2 | 3 |
| 15/09/2007 | 10.6 | 61 | 6 | SSE | 2 | 0 | 16 | 39 | 6 | NNW | 2 | 0 | 0 |
| 16/09/2007 | 14 | 49 | 1 | NNE | 2 | 0 | 22.9 | 29 | 0 | NNW | 2 | 0 | 0 |
| 17/09/2007 | 15.5 | 79 | 7 | NW | 7 | 0 | 16.8 | 57 | 5 | WNW | 9 | 0 | 0 |
| 18/09/2007 | 14.7 | 61 | 7 | NW | 22 | 0 | 17.8 | 55 | 5 | W | 28 | 0 | 0 |
| 19/09/2007 | 13.8 | 74 | 7 | WSW | 11 | 1 | 17.5 | 49 | 6 | W | 19 | 0 | 0 |
| 20/09/2007 | 12.5 | 93 | 8 | NW | 2 | 0 | 18 | 50 | 7 | NW | 19 | 0 | 0 |
| 21/09/2007 | 13.4 | 81 | 8 | NW | 15 | 0 | 9.5 | 73 | 6 | S | 19 | 0 | 0 |
| 22/09/2007 | 8.6 | 66 | 1 | S | 2 | 0 | 13.4 | 48 | 4 | S | 2 | 0 | 0 |
| 23/09/2007 | 12 | 57 | 4 | ENE | 15 | 0 | 18.9 | 26 | 8 | NE | 11 | 0 | 0 |
| 24/09/2007 | 20 | 26 | 1 | NNW | 15 | 0 | 26 | 29 | 2 | NNW | 28 | 0 | 0 |
| 25/09/2007 | 11.6 | 93 | 8 | S | 9 | 2 | 10.9 | 95 | 8 | S | 2 | 0 | 0 |
| 26/09/2007 | 9.3 | 95 | 8 | S | 9 | 2 | 13 | 84 | 7 | SSE | 7 | 0 | 1 |
| 27/09/2007 | 11 | 64 | 7 | NW | 11 | 0 | 15 | 55 | 7 | NW | 22 | 0 | 0 |
| 28/09/2007 | 13.5 | 82 | 7 | NNW | 9 | 0 | 20 | 51 | 5 | WNW | 4 | 0 | 0 |
| 29/09/2007 | 15.5 | 73 | 0 | NE | 2 | 0 | 23.2 | 37 | 1 | NNW | 15 | 0 | 0 |
| 30/09/2007 | 14.6 | 72 | 6 | WNW | 4 | 0 | 18 | 53 | 7 | W | 7 | 1 | 1 |
| 1/10/2007 | 14.6 | 75 | 7 | NW | 19 | 0 | 18.2 | 64 | 7 | NW | 22 | 0 | 0 |
| 2/10/2007 | 11.8 | 73 | 8 | SSW | 15 | 3 | 16 | 46 | 4 | SSW | 11 | 2 | 4 |
| 3/10/2007 | 12 | 54 | 3 | S | 2 | 0 | 15.9 | 43 | 7 | SW | 7 | 2 | 3 |
| 4/10/2007 | 12.8 | 79 | 7 | NW | 9 | 0 | 10.8 | 75 | 6 | SW | 9 | 3 | 2 |
| 5/10/2007 | 10.6 | 64 | 6 | S | 11 | 1 | 14 | 48 | 7 | SSE | 15 | 0 | 0 |
| 6/10/2007 | 11.2 | 69 | 7 | SSE | 4 | 0 | 14 | 51 | 7 | SSE | 7 | 0 | 0 |
| 7/10/2007 | 12 | 74 | 5 | SE | 7 | 0 | 18.5 | 37 | 1 | SE | 2 | 0 | 0 |
| 8/10/2007 | 15 | 60 | 0 | ENE | 9 | 0 | 23 | 26 | 0 | N | 4 | 0 | 0 |
| 9/10/2007 | 15.5 | 63 | 1 | SW | 9 | 2 | 21.3 | 36 | 3 | SSE | 2 | 0 | 0 |
| 10/10/2007 | 14.7 | 65 | 0 | ENE | 4 | 0 | 23.5 | 31 | 3 | NW | 2 | 0 | 0 |
| 11/10/2007 | 13 | 73 | 6 | SW | 11 | 2 | 17.9 | 43 | 7 | SW | 11 | 3 | 4 |
| 12/10/2007 | 13.2 | 65 | 6 | SSW | 9 | 1 | 17 | 39 | 7 | SSW | 9 | 1 | 1 |
| 13/10/2007 | 13.4 | 56 | 1 | S | 4 | 0 | 19.1 | 35 | 4 | SSE | 9 | 0 | 0 |
| 14/10/2007 | 16.4 | 52 | 0 | SE | 15 | 0 | 25.4 | 27 | 0 | NE | 4 | 0 | 0 |
| 15/10/2007 | 21.6 | 35 | 5 | NNE | 19 | 0 | 32.9 | 11 | 5 | NNW | 37 | 0 | 0 |
| 16/10/2007 | 15.8 | 79 | 8 | SW | 7 | 2 | 20.6 | 53 | 6 | SSW | 7 | 1 | 1 |
| 17/10/2007 | 13.4 | 70 | 8 | SW | 4 | 1 | 22.8 | 33 | 0 | SSW | 4 | 1 | 1 |
| 18/10/2007 | 17.8 | 63 | 7 | SSW | 15 | 0 | 24.5 | 31 | 3 | S | 2 | 0 | 0 |
| 19/10/2007 | 18.6 | 61 | 0 | ESE | 4 | 0 | 29.5 | 26 | 4 | SSE | 4 | 0 | 0 |
| 20/10/2007 | 20.4 | 50 | 6 | NE | 33 | 0 | 29.4 | 25 | 7 | N | 15 | 0 | 0 |
| 21/10/2007 | 24.5 | 41 | 7 | NNE | 22 | 0 | 30 | 29 | 4 | N | 11 | 0 | 0 |
| 22/10/2007 | 23.3 | 49 | 4 | NNW | 4 | 0 | 28.9 | 31 | 8 | WNW | 4 | 0 | 0 |
| 23/10/2007 | 22.2 | 57 | 7 | E | 2 | 0 | 28.9 | 37 | 8 | N | 15 | 0 | 0 |
| 24/10/2007 | 17.5 | 85 | 8 | NW | 7 | 0 | 19 | 73 | 7 | NNW | 22 | 0 | 0 |
| 25/10/2007 | 15.2 | 62 | 5 | WSW | 15 | 0 | 19.7 | 40 | 5 | SW | 2 | 0 | 0 |
| 26/10/2007 | 12.5 | 72 | 7 | Calm | 0 | 0 | 22 | 37 | 0 | NW | 7 | 0 | 0 |
| 27/10/2007 | 13.4 | 79 | 1 | S | 4 | 1 | 23.9 | 32 | 3 | SE | 7 | 0 | 0 |
| 28/10/2007 | 18 | 65 | 5 | SE | 26 | 0 | 24.5 | 37 | 6 | SE | 15 | 0 | 0 |
| 29/10/2007 | 15.5 | 79 | 7 | SSW | 19 | 0 | 17.9 | 73 | 7 | SW | 15 | 2 | 0 |
| 30/10/2007 | 12 | 65 | 5 | SSE | 7 | 0 | 10.8 | 86 | 8 | W | 4 | 1 | 1 |
| 31/10/2007 | 12.4 | 74 | 7 | SE | 15 | 0 | 17 | 52 | 7 | SE | 2 | 0 | 0 |
| 1/11/2007 | 12.8 | 67 | 0 | S | 4 | 0 | 20 | 30 | 4 | SE | 8 | 1 | 1 |
| 2/11/2007 | 11.5 | 59 | 6 | ESE | 15 | 0 | 18.9 | 28 | 0 | ESE | 4 | 0 | 0 |
| 3/11/2007 | 11.8 | 51 | 0 | SE | 11 | 1 | 21.1 | 18 | 0 | ENE | 4 | 0 | 0 |
| 4/11/2007 | 14 | 45 | 0 | NE | 15 | 0 | 24.7 | 17 | 0 | E | 9 | 0 | 0 |
| 5/11/2007 | 18 | 24 | 2 | NNE | 18 | 0 | 29.5 | 9 | 3 | NE | 11 | 0 | 0 |
| 6/11/2007 | 15 | 80 | 4 | W | 4 | 0 | 24.5 | 34 | 4 | SW | 9 | 3 | 2 |
| 7/11/2007 | 16.1 | 66 | 3 | SSW | 4 | 0 | 24.9 | 25 | 0 | Calm | 0 | 0 | 0 |
| 8/11/2007 | 17.2 | 66 | 7 | SSE | 4 | 0 | 25.8 | 29 | 2 | SSW | 9 | 4 | 2 |
| 9/11/2007 | 15 | 68 | 1 | SE | 4 | 0 | 24.8 | 28 | 1 | NE | 2 | 0 | 0 |
| 10/11/2007 | 15.5 | 61 | 1 | SE | 9 | 1 | 26.4 | 26 | 1 | S | 11 | 1 | 0 |
| 11/11/2007 | 17.8 | 60 | 0 | SE | 8 | 1 | 30.5 | 25 | 0 | SE | 4 | 0 | 0 |
| 12/11/2007 | 23.5 | 41 | 1 | NE | 4 | 0 | 36.3 | 12 | 5 | NNE | 9 | 0 | 0 |
| 13/11/2007 | 24.3 | 47 | 2 | SSE | 4 | 0 | 33.5 | 25 | 4 | SW | 8 | 3 | 4 |
| 14/11/2007 | 17.2 | 73 | 2 | SSE | 9 | 1 | 31.9 | 26 | 0 | WNW | 4 | 0 | 0 |
| 15/11/2007 | 17.9 | 59 | 1 | SSE | 4 | 0 | 29.6 | 13 | 3 | WSW | 4 | 1 | 0 |
| 16/11/2007 | 16.6 | 60 | 1 | SE | 4 | 0 | 27 | 24 | 0 | SSE | 4 | 0 | 0 |
| 17/11/2007 | 19 | 51 | 0 | NNW | 15 | 0 | 30.5 | 12 | 0 | NW | 22 | 0 | 0 |
| 18/11/2007 | 15 | 58 | 5 | SW | 8 | 2 | 22.3 | 26 | 0 | WSW | 4 | 1 | 2 |
| 19/11/2007 | 16.5 | 52 | 6 | WNW | 15 | 0 | 19 | 31 | 6 | WSW | 15 | 2 | 0 |
| 20/11/2007 | 12.4 | 55 | 3 | S | 8 | 1 | 20.8 | 25 | 0 | SSW | 9 | 3 | 1 |
| 21/11/2007 | 17 | 44 | 0 | NE | 2 | 0 | 26.5 | 18 | 0 | SE | 2 | 0 | 0 |
| 22/11/2007 | 20.2 | 40 | 0 | Calm | 0 | 0 | 31.8 | 10 | 1 | NE | 2 | 0 | 0 |
| 23/11/2007 | 24.4 | 23 | 3 | NE | 2 | 0 | 34.7 | 8 | 3 | E | 2 | 0 | 0 |
| 24/11/2007 | 19.8 | 61 | 0 | SE | 2 | 0 | 36 | 7 | 0 | SE | 2 | 0 | 0 |
| 25/11/2007 | 19.5 | 62 | 0 | S | 2 | 0 | 33 | 19 | 0 | WSW | 11 | 1 | 0 |
| 26/11/2007 | 16.2 | 63 | 0 | S | 11 | 1 | 27.5 | 18 | 0 | S | 9 | 2 | 1 |
| 27/11/2007 | 15.8 | 54 | 1 | SE | 4 | 0 | 25.4 | 24 | 0 | SSE | 2 | 0 | 0 |
| 28/11/2007 | 16.2 | 59 | 7 | ENE | 4 | 0 | 31 | 16 | 2 | N | 2 | 0 | 0 |
| 29/11/2007 | 18.5 | 69 | 2 | SW | 4 | 1 | 27.2 | 32 | 3 | NW | 15 | 0 | 0 |
| 30/11/2007 | 17 | 67 | 6 | NW | 15 | 0 | 21 | 32 | 4 | NW | 28 | 0 | 0 |
| 1/12/2007 | 14.5 | 77 | 8 | SW | 11 | 2 | 19.9 | 41 | 7 | SW | 11 | 2 | 0 |
| 2/12/2007 | 14.8 | 52 | 5 | S | 11 | 1 | 22.5 | 26 | 1 | SW | 2 | 0 | 0 |
| 3/12/2007 | 15.8 | 57 | 5 | SW | 2 | 0 | 23.2 | 28 | 2 | SW | 2 | 0 | 0 |
| 4/12/2007 | 16.5 | 60 | 1 | ESE | 2 | 0 | 25.5 | 24 | 5 | N | 2 | 0 | 0 |
| 5/12/2007 | 14 | 75 | 8 | SW | 4 | 2 | 20 | 33 | 7 | WNW | 15 | 0 | 0 |
| 6/12/2007 | 10.9 | 61 | 3 | SSW | 8 | 3 | 18.3 | 32 | 4 | SSW | 4 | 1 | 1 |
| 7/12/2007 | 14.4 | 57 | 4 | S | 4 | 0 | 22.5 | 36 | 2 | WSW | 8 | 2 | 1 |
| 8/12/2007 | 13.8 | 66 | 7 | SSE | 11 | 0 | 21.7 | 26 | 1 | SSW | 2 | 0 | 0 |
| 9/12/2007 | 13.4 | 53 | 0 | ESE | 15 | 0 | 23.6 | 22 | 1 | SE | 11 | 0 | 1 |
| 10/12/2007 | 16 | 51 | 0 | SE | 9 | 1 | 28.8 | 18 | 0 | SE | 4 | 0 | 0 |
| 11/12/2007 | 21.5 | 33 | 7 | NE | 4 | 0 | 30.5 | 14 | 7 | SE | 8 | 1 | 2 |
| 12/12/2007 | 21 | 44 | 3 | NNW | 4 | 0 | 28 | 34 | 5 | SW | 22 | 0 | 0 |
| 13/12/2007 | 16 | 65 | 3 | SW | 4 | 0 | 24 | 30 | 4 | N | 4 | 0 | 0 |
| 14/12/2007 | 15.2 | 52 | 5 | SE | 18 | 0 | 18.5 | 44 | 7 | SE | 22 | 0 | 0 |
| 15/12/2007 | 13.9 | 58 | 8 | ESE | 9 | 2 |  |  |  |  |  |  |  |
| 16/12/2007 | 11.8 | 95 | 8 | ESE | 11 | 0 | 13.5 | 91 | 8 | ESE | 8 | 0 | 0 |
| 17/12/2007 | 15.8 | 85 | 8 | Calm | 0 | 0 | 19.7 | 68 | 7 | NNW | 4 | 0 | 0 |
| 18/12/2007 | 17.6 | 70 | 7 | SSW | 2 | 0 | 24.1 | 40 | 6 | NNW | 2 | 0 | 0 |
| 19/12/2007 | 16.7 | 82 | 7 | Calm | 0 | 0 | 24.1 | 41 | 5 | SSE | 4 | 0 | 0 |
| 20/12/2007 | 15.5 | 77 | 7 | SE | 2 | 0 | 23.2 | 35 | 7 | SW | 9 | 2 | 4 |
| 21/12/2007 | 14.1 | 58 | 7 | SSE | 8 | 1 | 22.5 | 28 | 1 | SSE | 4 | 0 | 0 |
| 22/12/2007 | 14.4 | 60 | 3 | SE | 11 | 1 | 24 | 32 | 8 | S | 2 | 0 | 0 |
| 23/12/2007 | 17 | 61 | 1 | SSE | 11 | 0 | 28.6 | 24 | 1 | E | 2 | 0 | 0 |
| 24/12/2007 | 19.9 | 44 | 0 | ENE | 11 | 0 | 30 | 17 | 0 | E | 8 | 0 | 0 |
| 25/12/2007 | 25 | 26 | 0 | NE | 15 | 0 | 34.9 | 10 | 0 | N | 4 | 0 | 0 |
| 26/12/2007 | 31.9 | 12 | 0 | N | 22 | 0 | 40.8 | 9 | 2 | NNW | 4 | 0 | 0 |
| 27/12/2007 | 32 | 25 | 1 | NNW | 4 | 0 | 39 | 15 | 1 | W | 4 | 0 | 0 |
| 28/12/2007 | 18.6 | 62 | 5 | S | 11 | 1 | 28 | 28 | 0 | SSW | 4 | 1 | 0 |
| 29/12/2007 | 16.9 | 59 | 1 | SE | 15 | 1 | 28.7 | 36 | 0 | SSE | 4 | 0 | 0 |
| 30/12/2007 | 19.5 | 66 | 0 | SE | 2 | 0 | 32.2 | 29 | 0 | SSW | 11 | 1 | 1 |
| 31/12/2007 | 17.4 | 55 | 2 | SSE | 9 | 0 | 26.5 | 23 | 0 | SE | 17 | 0 | 0 |
| 1/01/2008 | 16.4 | 60 | 3 | SE | 5 | 1 | 29.5 | 18 | 6 | SSE | 9 | 1 | 1 |
| 2/01/2008 | 19.7 | 58 | 8 | ESE | 2 | 0 | 28.2 | 36 | 8 | ESE | 2 | 0 | 0 |
| 3/01/2008 | 22.5 | 55 | 7 | Calm | 0 | 0 | 32.5 | 40 | 8 | Calm | 0 | 0 | 0 |
| 4/01/2008 | 18.1 | 77 | 7 | S | 8 | 1 | 29.5 | 37 | 2 | SSE | 4 | 0 | 0 |
| 5/01/2008 | 14.8 | 65 | 2 | S | 8 | 1 | 27 | 27 | 0 | SSE | 8 | 0 | 0 |
| 6/01/2008 | 16.7 | 60 | 0 | SE | 8 | 1 | 28.7 | 22 | 3 | E | 8 | 0 | 0 |
| 7/01/2008 | 18.2 | 53 | 1 | ESE | 11 | 0 | 35.1 | 19 | 0 | SE | 5 | 0 | 0 |
| 8/01/2008 | 29 | 33 | 5 | NE | 11 | 0 | 38.8 | 19 | 7 | NW | 33 | 0 | 0 |
| 9/01/2008 | 19.8 | 64 | 1 | W | 4 | 0 | 28.6 | 32 | 0 | SW | 4 | 3 | 0 |
| 10/01/2008 | 16.1 | 59 | 4 | SSW | 15 | 3 | 24 | 26 | 1 | SW | 4 | 3 | 0 |
| 11/01/2008 | 16.7 | 56 | 0 | SE | 2 | 0 | 25.6 | 24 | 0 | SSW | 4 | 1 | 0 |
| 12/01/2008 | 17.5 | 61 | 0 | SSE | 4 | 0 | 28.5 | 29 | 1 | SSE | 2 | 0 | 0 |
| 13/01/2008 | 17 | 66 | 5 | ESE | 4 | 0 | 29 | 29 | 0 | Calm | 0 | 0 | 0 |
| 14/01/2008 | 21.8 | 46 | 0 | NE | 2 | 0 | 35.6 | 17 | 0 | WNW | 2 | 0 | 0 |
| 15/01/2008 | 18 | 62 | 3 | SSE | 18 | 0 | 28.4 | 22 | 0 | SSE | 22 | 0 | 0 |
| 16/01/2008 | 17.7 | 55 | 0 | SE | 22 | 0 | 30.5 | 27 | 0 | SE | 15 | 1 | 0 |
| 17/01/2008 | 24 | 43 | 1 | ENE | 15 | 0 | 36.6 | 19 | 2 | NNE | 2 | 0 | 0 |
| 18/01/2008 | 23 | 52 | 0 | NW | 4 | 0 | 32.9 | 32 | 1 | SSW | 11 | 2 | 2 |
| 19/01/2008 | 17.2 | 92 | 8 | S | 11 | 1 | 23.6 | 52 | 7 | SSE | 11 | 1 | 1 |
| 20/01/2008 | 17.2 | 78 | 7 | SE | 9 | 1 | 25.6 | 45 | 4 | SE | 8 | 1 | 0 |
| 21/01/2008 | 19 | 63 | 6 | ESE | 11 | 0 | 25.3 | 40 | 7 | ESE | 15 | 0 | 0 |
| 22/01/2008 | 19.4 | 56 | 0 | ESE | 8 | 0 | 27.2 | 34 | 1 | SE | 4 | 0 | 0 |
| 23/01/2008 | 18.2 | 68 | 0 | SE | 4 | 0 | 30.9 | 26 | 1 | ESE | 2 | 0 | 0 |
| 24/01/2008 | 21.5 | 53 | 0 | NE | 2 | 0 | 34.5 | 22 | 5 | NE | 2 | 0 | 0 |
| 25/01/2008 | 20.9 | 71 | 2 | SE | 2 | 0 | 33.4 | 29 | 4 | Calm | 0 | 0 | 0 |
| 26/01/2008 | 20.7 | 70 | 0 | SE | 2 | 0 | 34.2 | 26 | 0 | SSE | 8 | 0 | 1 |
| 27/01/2008 | 17.9 | 68 | 1 | SSE | 9 | 1 | 33.5 | 25 | 0 | SSE | 9 | 1 | 1 |
| 28/01/2008 | 17.6 | 71 | 1 | SSE | 8 | 1 | 30 | 30 | 0 | SE | 8 | 1 | 0 |
| 29/01/2008 | 18.6 | 64 | 0 | SSE | 11 | 0 | 30 | 29 | 0 | SE | 11 | 0 | 1 |
| 30/01/2008 | 17.4 | 70 | 8 | SE | 15 | 0 | 25 | 41 | 4 | ESE | 15 | 0 | 0 |
| 31/01/2008 | 16.1 | 74 | 8 | SE | 11 | 1 | 25.7 | 41 | 0 | SE | 8 | 1 | 1 |
| 1/02/2008 | 19 | 72 | 7 | SE | 8 | 1 | 29.2 | 32 | 1 | SE | 4 | 0 | 0 |
| 2/02/2008 | 20.1 | 76 | 7 | SE | 8 | 1 | 29 | 41 | 5 | SSE | 4 | 0 | 0 |
| 3/02/2008 | 20.5 | 76 | 0 | SSE | 2 | 0 | 33.5 | 30 | 5 | ESE | 4 | 0 | 0 |
| 4/02/2008 | 20 | 77 | 7 | SE | 9 | 1 | 29 | 43 | 5 | S | 15 | 2 | 0 |
| 5/02/2008 | 22.1 | 58 | 6 | SE | 2 | 0 | 32.7 | 26 | 6 | ENE | 4 | 0 | 0 |
| 6/02/2008 | 23 | 54 | 2 | SE | 2 | 0 | 32.3 | 22 | 4 | E | 15 | 0 | 0 |
| 7/02/2008 | 19 | 63 | 7 | SSE | 8 | 0 | 27.1 | 36 | 8 | ENE | 4 | 0 | 0 |
| 8/02/2008 | 17.7 | 91 | 7 | Calm | 0 | 0 | 24.4 | 54 | 8 | NE | 4 | 0 | 0 |
| 9/02/2008 | 19.7 | 67 | 6 | ESE | 8 | 0 | 27 | 38 | 6 | ESE | 4 | 0 | 0 |
| 10/02/2008 | 19.6 | 61 | 0 | E | 4 | 0 | 30.5 | 28 | 1 | ENE | 4 | 0 | 0 |
| 11/02/2008 | 20.8 | 61 | 6 | Calm | 0 | 0 | 33.3 | 24 | 5 | ENE | 2 | 0 | 0 |
| 12/02/2008 | 19.3 | 61 | 3 | ESE | 11 | 0 | 30.1 | 27 | 1 | ESE | 11 | 0 | 0 |
| 13/02/2008 | 19.6 | 49 | 0 | NE | 18 | 0 | 30.4 | 25 | 0 | NE | 4 | 0 | 0 |
| 14/02/2008 | 22.8 | 38 | 4 | NNE | 8 | 0 | 36 | 17 | 5 | NNE | 15 | 0 | 0 |
| 15/02/2008 | 21.2 | 68 | 6 | SSW | 4 | 1 | 30.3 | 34 | 4 | SW | 4 | 1 | 3 |
| 16/02/2008 | 19.4 | 78 | 8 | SW | 2 | 0 | 29 | 33 | 0 | Calm | 0 | 0 | 0 |
| 17/02/2008 | 20 | 74 | 1 | SW | 4 | 0 | 27.8 | 38 | 1 | NW | 4 | 0 | 0 |
| 18/02/2008 | 16.5 | 84 | 7 | SSW | 4 | 2 | 25 | 25 | 0 | SW | 8 | 2 | 3 |
| 19/02/2008 | 14.5 | 63 | 3 | SSE | 11 | 0 | 22.5 | 24 | 2 | SE | 15 | 0 | 0 |
| 20/02/2008 | 14.2 | 63 | 2 | ESE | 8 | 0 | 22.4 | 28 | 6 | NE | 9 | 0 | 0 |
| 21/02/2008 | 15.1 | 68 | 2 | SE | 2 | 0 | 24 | 28 | 1 | SE | 9 | 1 | 1 |
| 22/02/2008 | 15 | 49 | 6 | SE | 11 | 0 | 22.6 | 33 | 0 | SE | 15 | 1 | 0 |
| 23/02/2008 | 14 | 61 | 1 | ESE | 15 | 1 | 24.3 | 26 | 0 | ESE | 4 | 0 | 0 |
| 24/02/2008 | 16 | 71 | 0 | E | 5 | 0 | 27.5 | 26 | 0 | E | 2 | 0 | 0 |
| 25/02/2008 | 20 | 52 | 0 | NE | 2 | 0 | 30.1 | 21 | 0 | NE | 2 | 0 | 0 |
| 26/02/2008 | 22.2 | 48 | 0 | NE | 2 | 0 | 35.6 | 16 | 1 | N | 5 | 0 | 0 |
| 27/02/2008 | 18.5 | 72 | 5 | E | 11 | 0 | 29.7 | 35 | 2 | SE | 9 | 1 | 1 |
| 28/02/2008 | 22.2 | 55 | 4 | ENE | 4 | 0 | 37.2 | 17 | 1 | NNW | 8 | 0 | 0 |
| 29/02/2008 | 27.5 | 28 | 1 | NE | 2 | 0 | 39 | 14 | 1 | NNW | 9 | 0 | 0 |
| 1/03/2008 | 19.9 | 92 |  | Calm | 0 | 0 | 35.5 | 21 | 0 | NW | 9 | 0 | 0 |
| 2/03/2008 | 19.5 | 77 | 8 | ESE | 2 | 0 | 27.4 | 44 | 0 | SE | 9 | 1 | 1 |
| 3/03/2008 | 16.7 | 75 | 7 | SE | 11 | 1 | 26.9 | 36 | 0 | ENE | 8 | 0 | 0 |
| 4/03/2008 | 17.8 | 77 | 2 | ENE | 2 | 0 | 32.1 | 31 | 3 | Calm | 0 | 0 | 0 |
| 5/03/2008 | 19.5 | 77 | 2 | S | 4 | 0 | 33 | 30 | 6 | S | 2 | 0 | 0 |
| 6/03/2008 | 22 | 78 | 8 | Calm | 0 | 0 | 29.5 | 39 | 7 | S | 4 | 0 | 2 |
| 7/03/2008 | 25.2 | 40 | 8 | Calm | 0 | 0 | 25.5 | 58 | 7 | Calm | 0 | 0 | 0 |
| 8/03/2008 | 16.4 | 81 | 1 | NNE | 4 | 0 | 25 | 50 | 6 | WNW | 8 | 0 | 0 |
| 9/03/2008 | 16.6 | 78 | 1 | SE | 4 | 0 | 27 | 36 | 1 | SE | 4 | 0 | 0 |
| 10/03/2008 | 17 | 75 | 8 | SE | 2 | 0 | 28.6 | 38 | 2 | ESE | 9 | 0 | 0 |
| 11/03/2008 | 18.5 | 79 | 2 | SE | 4 | 0 | 32 | 32 | 1 | Calm | 0 | 0 | 0 |
| 12/03/2008 | 19 | 82 | 1 | S | 2 | 0 | 34.5 | 29 | 0 | Calm | 0 | 0 | 0 |
| 13/03/2008 | 18 | 68 | 7 | NW | 15 | 0 | 21.6 | 37 | 5 | WSW | 28 | 0 | 1 |
| 14/03/2008 | 13.6 | 63 | 0 | Calm | 0 | 0 | 22.5 | 28 | 0 | Calm | 0 | 0 | 0 |
| 15/03/2008 | 13.9 | 80 | 1 | Calm | 0 | 0 | 26.4 | 31 | 0 | NE | 4 | 0 | 0 |
| 16/03/2008 | 16 | 82 | 0 | Calm | 0 | 0 | 25.5 | 36 | 4 | NW | 4 | 0 | 0 |
| 17/03/2008 | 17.3 | 82 | 3 | SE | 2 | 0 | 26 | 32 | 5 | SSE | 11 | 0 | 1 |
| 18/03/2008 | 16.3 | 81 | 7 | ESE | 2 | 0 | 25.5 | 37 | 1 | S | 15 | 2 | 1 |
| 19/03/2008 | 15.6 | 78 | 0 | Calm | 0 | 0 | 28 | 36 | 5 | SSE | 4 | 0 | 0 |
| 20/03/2008 | 15 | 71 | 1 | SE | 5 | 1 | 25.5 | 24 | 1 | SE | 11 | 0 | 1 |
| 21/03/2008 | 15.5 | 74 | 7 | E | 2 | 0 | 29 | 31 | 3 | Calm | 0 | 0 | 0 |
| 22/03/2008 | 17 | 76 | 8 | S | 2 | 0 | 25.3 | 50 | 3 | SSE | 2 | 0 | 0 |
| 23/03/2008 | 14 | 83 | 7 | S | 11 | 1 | 20.6 | 39 | 6 | SE | 9 | 1 | 2 |
| 24/03/2008 | 12 | 68 | 8 | ESE | 2 | 0 | 20.3 | 38 | 0 | SE | 9 | 1 | 1 |
| 25/03/2008 | 12 | 75 | 3 | SE | 4 | 0 | 21.5 | 33 | 4 | ESE | 9 | 0 | 0 |
| 26/03/2008 | 13.8 | 60 | 0 | E | 15 | 0 | 24.5 | 25 | 0 | ENE | 8 | 0 | 0 |
| 27/03/2008 | 16 | 55 | 0 | Calm | 0 | 0 | 27.1 | 23 | 1 | SE | 2 | 0 | 0 |
| 28/03/2008 | 14 | 62 | 5 | SSE | 2 | 0 | 26.6 | 22 | 5 | SE | 5 | 0 | 1 |
| 29/03/2008 | 13.5 | 83 | 5 | ESE | 2 | 0 | 22.5 | 31 | 0 | SSE | 4 | 0 | 0 |
| 30/03/2008 | 14.2 | 66 | 0 | SE | 11 | 0 | 23.5 | 25 | 1 | SE | 4 | 0 | 0 |
| 31/03/2008 | 14.9 | 62 | 8 | ESE | 11 | 0 | 14 | 96 | 8 | SSE | 2 | 0 | 0 |
| 1/04/2008 | 15.5 | 78 | 6 | S | 15 | 1 | 20 | 47 | 4 | S | 19 | 0 | 0 |
| 2/04/2008 | 13 | 61 | 7 | SE | 15 | 1 | 18.6 | 42 | 0 | SE | 11 | 0 | 1 |
| 3/04/2008 | 13.6 | 61 | 0 | ENE | 9 | 0 | 22 | 33 | 0 | ENE | 4 | 0 | 0 |
| 4/04/2008 | 19 | 39 | 1 | NNE | 2 | 0 | 26.8 | 25 | 4 |  | Calm | 0 | 0 |
| 5/04/2008 | 17.5 | 95 | 8 | NW | 4 | 0 | 18.5 | 92 | 7 | N | 2 | 0 | 0 |
| 6/04/2008 | 19 | 90 | 8 | WNW | 2 | 0 | 25.5 | 46 | 5 | NW | 4 | 0 | 0 |
| 7/04/2008 | 16.5 | 80 | 3 | NNW | 4 | 0 | 24.8 | 42 | 1 | WNW | 4 | 0 | 0 |
| 8/04/2008 | 17.8 | 73 | 6 | SSW | 4 | 0 | 22.5 | 41 | 1 | S | 2 | 0 | 0 |
| 9/04/2008 | 15.4 | 73 | 7 | N | 2 | 0 | 24.1 | 41 | 7 | NNW | 7 | 0 | 0 |
| 10/04/2008 | 17 | 82 | 8 | WSW | 11 | 1 | 20.5 | 38 | 1 | SW | 9 | 2 | 3 |
| 11/04/2008 | 13.5 | 68 | 6 | SSE | 4 | 0 | 19.8 | 38 | 1 | SE | 2 | 0 | 0 |
| 12/04/2008 | 15.6 | 63 | 0 | NE | 2 | 0 | 24.5 | 25 | 0 |  | Calm | 0 | 1 |
| 13/04/2008 | 17.9 | 44 | 0 |  | Calm | 0 | 26.2 | 23 | 1 | NE | 2 | 0 | 0 |
| 14/04/2008 | 21 | 30 | 0 | NNE | 9 | 0 | 29 | 13 | 1 | NNW | 15 | 1 | 0 |
| 15/04/2008 | 21 | 46 | 1 |  | Calm | 0 | 26.5 | 26 | 6 | SW | 9 | 1 | 3 |
| 16/04/2008 | 17 | 62 | 6 |  | Calm | 0 | 28.1 | 27 | 7 |  | Calm | 0 | 0 |
| 17/04/2008 | 14.4 | 91 | 8 |  | Calm | 0 | 16 | 83 | 8 | NNW | 4 | 0 | 0 |
| 18/04/2008 | 13 | 96 | 8 |  | Calm | 0 | 19.5 | 52 | 6 | WNW | 7 | 0 | 0 |
| 19/04/2008 | 10.5 | 69 | 3 | SSW | 4 | 0 | 14.2 | 46 | 6 | SSW | 4 | 2 | 1 |
| 20/04/2008 | 11.9 | 89 | 7 | WSW | 7 | 2 | 15.5 | 53 | 8 | SSW | 15 | 1 | 1 |
| 21/04/2008 | 10 | 93 | 8 | SSE | 4 | 0 | 15 | 61 | 5 | WSW | 4 | 1 | 1 |
| 22/04/2008 | 14 | 83 | 2 | NNW | 7 | 0 | 18.8 | 58 | 4 | WNW | 4 | 0 | 0 |
| 23/04/2008 | 15 | 88 | 8 | NW | 4 | 0 | 19 | 52 | 5 | W | 11 | 1 | 0 |
| 24/04/2008 | 13.9 | 68 | 0 |  | Calm | 0 | 18.8 | 44 | 1 | WSW | 2 | 0 | 0 |
| 25/04/2008 | 13.2 | 79 | 0 | ESE | 4 | 0 | 20 | 45 | 1 | SSE | 2 | 0 | 0 |
| 26/04/2008 | 13.6 | 79 | 8 | WSW | 4 | 1 | 18 | 58 | 7 |  | Calm | 0 | 0 |
| 27/04/2008 | 10.5 | 92 | 8 | NW | 2 | 0 | 13.8 | 79 | 8 | NNW | 2 | 0 | 0 |
| 28/04/2008 | 16.3 | 72 | 5 | NNE | 2 | 0 | 21.8 | 65 | 6 | NW | 2 | 0 | 0 |
| 29/04/2008 | 15 | 89 | 5 |  | Calm | 0 | 19.6 | 49 | 4 | WNW | 11 | 0 | 0 |
| 30/04/2008 | 15 | 64 | 2 | SW | 4 | 1 | 18.2 | 41 | 3 | WSW | 2 | 0 | 0 |
| 1/05/2008 | 13 | 78 | 0 |  | Calm | 0 | 19 | 39 | 1 | NW | 2 | 0 | 0 |
| 2/05/2008 | 15 | 55 | 1 | NNE | 4 | 0 | 22 | 30 | 7 | NW | 4 | 0 | 0 |
| 3/05/2008 | 14.2 | 84 | 6 | SW | 4 | 0 | 18.5 | 68 | 7 | SSW | 2 | 0 | 0 |
| 4/05/2008 | 10.9 | 95 | 5 |  | Calm | 0 | 18.2 | 52 | 1 | SSW | 4 | 1 | 0 |
| 5/05/2008 | 12.1 | 84 | 0 |  | Calm | 0 | 19.8 | 50 | 1 |  | Calm | 0 | 0 |
| 6/05/2008 | 13 | 83 | 1 |  | Calm | 0 | 24 | 41 | 1 |  | Calm | 0 | 0 |
| 7/05/2008 | 18.6 | 65 | 4 |  | Calm | 0 | 25.5 | 35 | 6 | NW | 2 | 0 | 0 |
| 8/05/2008 | 15.5 | 90 | 8 | WNW | 2 | 0 | 22.6 | 62 | 8 | SW | 2 | 0 | 0 |
| 9/05/2008 | 17.9 | 88 | 1 |  | Calm | 0 | 24.5 | 47 | 3 | W | 2 | 0 | 0 |
| 10/05/2008 | 17 | 82 | 7 |  | Calm | 0 | 17.8 | 78 | 8 | WNW | 4 | 0 | 0 |
| 11/05/2008 | 16 | 94 | 7 | WNW | 2 | 0 | 20.8 | 52 | 5 | WSW | 2 | 0 | 1 |
| 12/05/2008 | 13.9 | 91 | 0 |  | Calm | 0 | 21.7 | 42 | 3 | WNW | 9 | 0 | 0 |
| 13/05/2008 | 16 | 81 | 2 | NW | 7 | 0 | 13 | 88 | 8 | WNW | 4 | 0 | 0 |
| 14/05/2008 | 10 | 86 | 1 | WSW | 2 | 0 | 14 | 57 | 8 | SW | 2 | 0 | 0 |
| 15/05/2008 | 12.5 | 89 | 8 | NW | 4 | 0 | 17.6 | 73 | 7 | W | 2 | 0 | 0 |
| 16/05/2008 | 12 | 72 | 6 | S | 2 | 0 | 14.5 | 57 | 7 | SSW | 4 | 1 | 0 |
| 17/05/2008 | 10 | 91 | 8 |  | Calm | 0 | 15.5 | 69 | 7 | S | 2 | 0 | 0 |
| 18/05/2008 | 11.1 | 89 | 0 |  | Calm | 0 | 19.2 | 47 | 1 | NW | 2 | 0 | 0 |
| 19/05/2008 | 11.8 | 97 |  |  | Calm | 0 | 18.5 | 55 | 5 | SW | 2 | 0 | 0 |
| 20/05/2008 | 11.6 | 93 | 7 | SE | 2 | 0 | 19.1 | 46 | 6 | N | 2 | 0 | 0 |
| 21/05/2008 | 9.8 | 84 | 0 |  | Calm | 0 | 20.5 | 36 | 0 | NW | 2 | 0 | 0 |
| 22/05/2008 | 13.5 | 89 | 6 | NW | 2 | 0 | 13.8 | 62 | 3 | NW | 7 | 0 | 0 |
| 23/05/2008 | 11.7 | 94 | 8 | WNW | 4 | 0 | 16 | 71 | 6 | W | 4 | 1 | 0 |
| 24/05/2008 | 13 | 88 | 8 | NNW | 7 | 0 | 16 | 86 | 8 | NW | 7 | 0 | 0 |
| 25/05/2008 | 12.8 | 91 | 5 |  | Calm | 0 | 16.3 | 65 | 7 |  | Calm | 0 | 0 |
| 26/05/2008 | 12.2 | 97 |  | N | 2 | 0 | 20 | 64 | 4 | N | 9 | 0 | 0 |
| 27/05/2008 | 15.5 | 70 | 4 | N | 2 | 0 | 21.8 | 38 | 7 | NNW | 11 | 0 | 0 |
| 28/05/2008 | 16.6 | 90 | 7 | NNW | 19 | 0 | 15.9 | 90 | 8 | W | 4 | 0 | 0 |
| 29/05/2008 | 12 | 94 | 1 |  | Calm | 0 | 16.9 | 59 | 5 | SSW | 2 | 0 | 0 |
| 30/05/2008 | 13.5 | 85 | 6 | NW | 2 | 0 | 20 | 61 | 6 | NNE | 2 | 0 | 0 |
| 31/05/2008 | 17.6 | 57 | 7 | NNW | 19 | 0 | 15.9 | 90 | 8 | NNW | 4 | 0 | 0 |
| 1/06/2008 | 14.8 | 99 | 4 |  | Calm | 0 | 18.5 | 75 | 6 | S | 2 | 0 | 0 |
| 2/06/2008 | 10.5 | 100 |  |  | Calm | 0 | 17 | 70 | 3 |  | Calm | 0 | 0 |
| 3/06/2008 | 10.4 | 91 | 0 |  | Calm | 0 | 16.7 | 73 | 2 | SSE | 2 | 0 | 0 |
| 4/06/2008 | 10.7 | 85 | 0 | SE | 2 | 0 | 15.9 | 56 | 1 | SE | 4 | 0 | 0 |
| 5/06/2008 | 11.5 | 88 | 7 | E | 2 | 0 | 17.2 | 60 | 4 |  | Calm | 0 | 0 |
| 6/06/2008 | 10.6 | 81 | 0 | ENE | 2 | 0 | 17.6 | 46 | 0 | NE | 2 | 0 | 0 |
| 7/06/2008 | 10 | 69 | 1 | N | 2 | 0 | 16.9 | 44 | 1 | NNW | 2 | 0 | 0 |
| 8/06/2008 | 10 | 66 | 5 | N | 15 | 0 | 17.9 | 37 | 7 | WNW | 15 | 0 | 0 |
| 9/06/2008 | 13 | 69 | 8 | NNW | 28 | 0 | 11.9 | 81 | 7 | SW | 7 | 3 | 2 |
| 10/06/2008 | 8 | 82 | 1 | NW | 2 | 0 | 12.5 | 77 | 7 | NW | 11 | 0 | 0 |
| 11/06/2008 | 10.9 | 94 | 8 | NW | 2 | 0 | 17.2 | 64 | 4 | WNW | 2 | 0 | 0 |
| 12/06/2008 | 10.1 | 99 | 1 | NW | 2 | 0 | 16.6 | 51 | 6 | SW | 2 | 0 | 0 |
| 13/06/2008 | 9.5 | 95 | 2 |  | Calm | 0 | 17.4 | 49 | 4 | NE | 2 | 0 | 0 |
| 14/06/2008 | 11.5 | 76 | 0 | NNW | 2 | 0 | 19 | 55 | 1 | NE | 4 | 0 | 0 |
| 15/06/2008 | 14 | 78 | 7 | NNW | 22 | 0 | 13.5 | 81 | 5 | NNW | 22 | 0 | 0 |
| 16/06/2008 | 7.5 | 86 | 1 | NW | 2 | 0 | 12.8 | 58 | 5 | W | 7 | 0 | 0 |
| 17/06/2008 | 10.4 | 95 | 7 | WNW | 2 | 0 | 13.5 | 52 | 6 | SSW | 2 | 0 | 0 |
| 18/06/2008 | 6.7 | 85 | 0 |  | Calm | 0 | 14 | 52 | 2 |  | Calm | 0 | 0 |
| 19/06/2008 | 5.3 | 92 | 0 |  | Calm | 0 | 15.5 | 53 | 1 |  | Calm | 0 | 0 |
| 20/06/2008 | 5.7 | 89 | 0 |  | Calm | 0 | 16.4 | 43 | 0 |  | Calm | 0 | 0 |
| 21/06/2008 | 10 | 69 | 0 | NE | 2 | 0 | 16.5 | 43 | 0 | NNE | 2 | 0 | 0 |
| 22/06/2008 | 11 | 59 | 2 | NNW | 4 | 0 | 17.6 | 44 | 3 | NNE | 2 | 0 | 0 |
| 23/06/2008 | 9.4 | 58 | 0 | N | 9 | 0 | 18.1 | 37 | 0 | NNW | 6 | 0 | 0 |
| 24/06/2008 | 10.2 | 56 | 0 | N | 4 | 0 | 20 | 26 | 0 | NNW | 22 | 0 | 0 |
| 25/06/2008 | 10.2 | 70 | 0 | NW | 4 | 0 | 15.3 | 51 | 3 | WNW | 2 | 0 | 0 |
| 26/06/2008 | 6.1 | 99 | 6 |  | Calm | 0 | 11.4 | 71 | 8 |  | Calm | 0 | 0 |
| 27/06/2008 | 10.5 | 99 | 8 | N | 2 | 0 | 14 | 96 | 8 |  | Calm | 0 | 0 |
| 28/06/2008 | 8 | 88 | 2 | WNW | 2 | 0 | 12.6 | 59 | 5 | WNW | 15 | 0 | 0 |
| 29/06/2008 | 9.5 | 87 | 1 | WNW | 2 | 0 | 13.6 | 55 | 6 | WSW | 4 | 1 | 2 |
| 30/06/2008 | 8.6 | 92 | 2 |  | Calm | 0 | 14.9 | 60 | 5 | NW | 2 | 0 | 0 |
| 1/07/2008 | 9.5 | 97 | 8 |  | Calm | 0 | 13.8 | 70 | 7 |  | Calm | 0 | 0 |
| 2/07/2008 | 4.6 | 100 | 8 |  | Calm | 0 | 15.5 | 58 | 1 | NE | 2 | 0 | 0 |
| 3/07/2008 | 9.6 | 84 | 7 |  | Calm | 0 | 16.5 | 56 | 2 | WSW | 2 | 0 | 0 |
| 4/07/2008 | 11.5 | 88 | 2 | NNW | 4 | 0 | 16.4 | 58 | 3 | NW | 6 | 0 | 0 |
| 5/07/2008 | 9.1 | 92 | 7 | NW | 4 | 0 | 11.7 | 66 | 7 | SSE | 6 | 0 | 1 |
| 6/07/2008 | 2.5 | 83 | 0 |  | Calm | 0 | 11.5 | 55 | 2 |  | Calm | 0 | 0 |
| 7/07/2008 | 7 | 75 | 2 | N | 2 | 0 | 14.3 | 41 | 1 | NNE | 7 | 0 | 0 |
| 8/07/2008 | 7.2 | 56 | 7 | N | 4 | 0 | 14.3 | 48 | 5 | NNW | 7 | 0 | 0 |
| 9/07/2008 | 11.4 | 97 | 8 |  | Calm | 0 | 13.5 | 72 | 8 | SSE | 4 | 0 | 0 |
| 10/07/2008 | 8.5 | 92 | 8 | ENE | 2 | 0 | 12.5 | 87 | 8 | NNE | 4 | 0 | 0 |
| 11/07/2008 | 11.2 | 80 | 8 | W | 4 | 0 | 13.9 | 49 | 4 | WSW | 11 | 1 | 0 |
| 12/07/2008 | 8.7 | 83 | 6 | NNW | 2 | 0 | 12.9 | 59 | 7 | SSW | 6 | 1 | 0 |
| 13/07/2008 | 7.4 | 96 | 5 | N | 2 | 0 | 13.9 | 74 | 5 |  | Calm | 0 | 0 |
| 14/07/2008 | 11.6 | 86 | 7 | N | 2 | 0 | 15.6 | 50 | 4 | N | 7 | 0 | 0 |
| 15/07/2008 | 11.1 | 57 | 4 | N | 26 | 0 | 12.5 | 94 | 8 | NNW | 4 | 0 | 0 |
| 16/07/2008 | 8.9 | 89 | 2 | NW | 4 | 0 | 10.7 | 72 | 6 | SSW | 4 | 0 | 0 |
| 17/07/2008 | 7 | 96 | 5 |  | Calm | 0 | 11.7 | 77 | 8 | NNE | 2 | 0 | 0 |
| 18/07/2008 | 9.9 | 86 | 5 | NNW | 7 | 0 | 9.5 | 80 | 4 | WNW | 6 | 0 | 0 |
| 19/07/2008 | 6.4 | 79 | 3 | SSW | 4 | 1 | 11 | 64 | 7 | SSW | 4 | 0 | 1 |
| 20/07/2008 | 6.9 | 97 | 8 |  | Calm | 0 | 12.5 | 65 | 8 |  | Calm | 0 | 0 |
| 21/07/2008 | 9.8 | 90 | 8 |  | Calm | 0 | 12.5 | 61 | 8 |  | Calm | 0 | 0 |
| 22/07/2008 | 7.9 | 75 | 5 | NNE | 2 | 0 | 12.4 | 63 | 8 | NNE | 6 | 0 | 0 |
| 23/07/2008 | 11.3 | 62 | 8 | NNW | 30 | 0 | 12.4 | 88 | 8 | NNW | 15 | 0 | 0 |
| 24/07/2008 | 10.5 | 91 | 2 | NNW | 4 | 0 | 14.1 | 63 | 2 | WSW | 11 | 2 | 0 |
| 25/07/2008 | 5.5 | 100 |  |  | Calm | 0 | 14.3 | 55 | 8 | NNE | 2 | 0 | 0 |
| 26/07/2008 | 6.4 | 97 |  |  | Calm | 0 | 14.8 | 60 | 4 | S | 2 | 0 | 0 |
| 27/07/2008 | 11.1 | 92 | 8 | NNE | 19 | 0 | 14.5 | 94 | 8 | N | 19 | 0 | 1 |
| 28/07/2008 | 13.3 | 85 | 5 | NNW | 11 | 0 | 11.1 | 89 | 8 | NW | 11 | 0 | 0 |
| 29/07/2008 | 9.1 | 85 | 3 | NNW | 7 | 0 | 13.1 | 70 | 6 | NW | 22 | 0 | 0 |
| 30/07/2008 | 11.1 | 81 | 5 | NW | 19 | 0 | 10 | 69 | 5 | W | 15 | 0 | 0 |
| 31/07/2008 | 6.3 | 81 | 0 | NW | 7 | 0 | 11.6 | 62 | 3 | W | 7 | 0 | 0 |
| 1/08/2008 | 7 | 93 | 7 | N | 4 | 0 | 12 | 84 | 8 | NNW | 4 | 0 | 0 |
| 2/08/2008 | 10 | 75 | 0 | WSW | 15 | 3 | 12.9 | 47 | 6 | SW | 7 | 1 | 1 |
| 3/08/2008 | 8.4 | 81 | 1 | SW | 2 | 0 | 12.4 | 61 | 7 | SSW | 2 | 0 | 0 |
| 4/08/2008 | 7.4 | 94 | 1 |  | Calm | 0 | 14 | 52 | 4 | NNW | 4 | 0 | 0 |
| 5/08/2008 | 9.9 | 82 | 1 |  | Calm | 0 | 14 | 62 | 5 |  | Calm | 0 | 0 |
| 6/08/2008 | 9 | 84 | 7 |  | Calm | 0 | 16.2 | 47 | 4 | WNW | 2 | 0 | 0 |
| 7/08/2008 | 9.8 | 90 | 8 | S | 2 | 0 | 12.5 | 66 | 8 | SE | 2 | 0 | 0 |
| 8/08/2008 | 7.7 | 84 | 0 | S | 2 | 0 | 14 | 43 | 5 | ESE | 4 | 0 | 0 |
| 9/08/2008 | 7 | 87 | 0 | SE | 2 | 0 | 13.8 | 46 | 4 | ESE | 4 | 0 | 0 |
| 10/08/2008 | 7.9 | 84 | 3 | SE | 2 | 0 | 13 | 55 | 6 | E | 2 | 0 | 0 |
| 11/08/2008 | 3.4 | 93 | 2 |  | Calm | 0 | 14.8 | 41 | 4 | SE | 4 | 0 | 0 |
| 12/08/2008 | 5.4 | 100 |  |  | Calm | 0 | 13.2 | 54 | 5 | ESE | 4 | 0 | 0 |
| 13/08/2008 | 6.6 | 91 | 1 | SE | 2 | 0 | 15.4 | 37 | 4 | ESE | 2 | 0 | 0 |
| 14/08/2008 | 9.5 | 62 | 0 |  | Calm | 0 | 16.4 | 31 | 3 | ENE | 2 | 0 | 0 |
| 15/08/2008 | 9.5 | 51 | 1 | NNE | 4 | 0 | 17.6 | 32 | 6 | NNE | 11 | 0 | 0 |
| 16/08/2008 | 11.8 | 43 | 0 | N | 9 | 0 | 18.2 | 34 | 2 | NNE | 4 | 0 | 0 |
| 17/08/2008 | 12 | 43 | 0 | N | 11 | 0 | 19 | 26 | 0 | NNE | 9 | 0 | 0 |
| 18/08/2008 | 11 | 48 | 0 | N | 7 | 0 | 19 | 26 | 2 | NNW | 11 | 0 | 1 |
| 19/08/2008 | 10.5 | 89 | 8 | NW | 4 | 0 | 11.8 | 86 | 7 |  | Calm | 0 | 0 |
| 20/08/2008 | 8.9 | 87 | 1 |  | Calm | 0 | 12.6 | 59 | 7 | SE | 7 | 1 | 1 |
| 21/08/2008 | 8.5 | 77 | 8 | ESE | 9 | 2 | 11.6 | 53 | 3 | ESE | 9 | 0 | 0 |
| 22/08/2008 | 8.4 | 67 | 1 | ENE | 19 | 0 | 14 | 45 | 4 | NE | 7 | 0 | 2 |
| 23/08/2008 | 9.6 | 60 | 1 | NNE | 11 | 0 | 16.4 | 33 | 0 | NNW | 7 | 0 | 0 |
| 24/08/2008 | 10.9 | 48 | 8 |  | Calm | 0 | 18.6 | 34 | 6 | NNW | 4 | 0 | 0 |
| 25/08/2008 | 12.2 | 93 | 7 | SSE | 4 | 0 | 15.1 | 50 | 6 | SSW | 4 | 0 | 1 |
| 26/08/2008 | 9.5 | 69 | 5 | SE | 4 | 0 | 16.3 | 49 | 1 | SE | 4 | 0 | 0 |
| 27/08/2008 | 10.5 | 73 | 4 | NE | 4 | 0 | 18.4 | 42 | 2 | NNE | 4 | 0 | 0 |
| 28/08/2008 | 12 | 88 | 5 |  | Calm | 0 | 17.9 | 55 | 6 | SW | 4 | 0 | 1 |
| 29/08/2008 | 12 | 82 | 7 | SSW | 2 | 0 | 12.7 | 74 | 6 | SSW | 7 | 0 | 1 |
| 30/08/2008 | 7.4 | 79 | 5 | SSW | 4 | 0 | 11 | 53 | 8 | SW | 4 | 0 | 1 |
| 31/08/2008 | 9.6 | 73 | 1 | NNW | 9 | 0 | 13 | 74 | 7 | NW | 15 | 0 | 0 |
| 1/09/2008 | 11.5 | 90 | 3 |  | Calm | 0 | 16.8 | 48 | 5 | NW | 2 | 0 | 0 |
| 2/09/2008 | 13.3 | 74 | 1 | NNE | 2 | 0 | 19.1 | 43 | 5 | N | 7 | 0 | 0 |
| 3/09/2008 | 15.1 | 62 | 0 | NW | 2 | 0 | 22 | 25 | 5 | N | 11 | 0 | 0 |
| 4/09/2008 | 11.8 | 87 | 8 | S | 4 | 0 | 18.9 | 51 | 5 | SSE | 22 | 0 | 0 |
| 5/09/2008 | 11.7 | 80 | 7 | S | 11 | 3 | 17.6 | 48 | 3 | SE | 7 | 0 | 1 |
| 6/09/2008 | 10.4 | 59 | 1 | SSE | 15 | 1 | 14.2 | 45 | 4 | SSE | 9 | 1 | 1 |
| 7/09/2008 | 10.1 | 68 | 3 | SE | 4 | 0 | 15.5 | 49 | 4 | NNW | 4 | 0 | 0 |
| 8/09/2008 | 8.9 | 85 | 7 |  | Calm | 0 | 17.9 | 39 | 7 | NNW | 11 | 0 | 0 |
| 9/09/2008 | 14 | 61 | 0 |  | Calm | 0 | 20.5 | 33 | 1 | WNW | 4 | 0 | 0 |
| 10/09/2008 | 13 | 73 | 4 | NNE | 2 | 0 |  |  |  |  |  |  |  |
| 11/09/2008 | 13.5 | 87 | 6 | WNW | 15 | 0 | 15.8 | 71 | 7 | WNW | 19 | 0 | 0 |
| 12/09/2008 | 14 | 68 | 8 | NW | 28 | 0 | 16.5 | 62 | 6 | NW | 37 | 0 | 0 |
| 13/09/2008 | 10.8 | 66 | 6 | WSW | 19 | 0 | 15.1 | 42 | 6 | WSW | 22 | 0 | 0 |
| 14/09/2008 | 13.2 | 79 | 7 | W | 19 | 0 | 17 | 35 | 5 | SW | 22 | 0 | 0 |
| 15/09/2008 | 10.6 | 61 | 6 | SSE | 2 | 0 | 16 | 39 | 6 | NNW | 2 | 0 | 0 |
| 16/09/2008 | 14 | 49 | 1 | NNE | 2 | 0 | 22.9 | 29 | 0 | NNW | 2 | 0 | 0 |
| 17/09/2008 | 15.5 | 79 | 7 | NW | 7 | 0 | 16.8 | 57 | 5 | WNW | 9 | 0 | 0 |
| 18/09/2008 | 14.7 | 61 | 7 | NW | 22 | 0 | 17.8 | 55 | 5 | W | 28 | 0 | 0 |
| 19/09/2008 | 13.8 | 74 | 7 | WSW | 11 | 2 | 17.5 | 49 | 6 | W | 19 | 0 | 0 |
| 20/09/2008 | 12.5 | 93 | 8 | NW | 2 | 0 | 18 | 50 | 7 | NW | 19 | 0 | 0 |
| 21/09/2008 | 13.4 | 81 | 8 | NW | 15 | 0 | 9.5 | 73 | 6 | S | 19 | 0 | 0 |
| 22/09/2008 | 8.6 | 66 | 1 | S | 2 | 0 | 13.4 | 48 | 4 | S | 2 | 0 | 0 |
| 23/09/2008 | 12 | 57 | 4 | ENE | 15 | 0 | 18.9 | 26 | 8 | NE | 11 | 0 | 0 |
| 24/09/2008 | 20 | 26 | 1 | NNW | 15 | 0 | 26 | 29 | 2 | NNW | 28 | 0 | 0 |
| 25/09/2008 | 11.6 | 93 | 8 | S | 9 | 1 | 10.9 | 95 | 8 | S | 2 | 0 | 0 |
| 26/09/2008 | 9.3 | 95 | 8 | S | 9 | 2 | 13 | 84 | 7 | SSE | 7 | 0 | 0 |
| 27/09/2008 | 11 | 64 | 7 | NW | 11 | 0 | 15 | 55 | 7 | NW | 22 | 0 | 0 |
| 28/09/2008 | 13.5 | 82 | 7 | NNW | 9 | 0 | 20 | 51 | 5 | WNW | 4 | 0 | 0 |
| 29/09/2008 | 15.5 | 73 | 0 | NE | 2 | 0 | 23.2 | 37 | 1 | NNW | 15 | 0 | 0 |
| 30/09/2008 | 14.6 | 72 | 6 | WNW | 4 | 0 | 18 | 53 | 7 | W | 7 | 0 | 0 |
| 1/10/2008 | 14.6 | 75 | 7 | NW | 19 | 0 | 18.2 | 64 | 7 | NW | 22 | 0 | 0 |
| 2/10/2008 | 11.8 | 73 | 8 | SSW | 15 | 1 | 16 | 46 | 4 | SSW | 11 | 3 | 2 |
| 3/10/2008 | 12 | 54 | 3 | S | 2 | 1 | 15.9 | 43 | 7 | SW | 7 | 3 | 0 |
| 4/10/2008 | 12.8 | 79 | 7 | NW | 9 | 0 | 10.8 | 75 | 6 | SW | 9 | 3 | 2 |
| 5/10/2008 | 10.6 | 64 | 6 | S | 11 | 1 | 14 | 48 | 7 | SSE | 15 | 2 | 0 |
| 6/10/2008 | 11.2 | 69 | 7 | SSE | 4 | 0 | 14 | 51 | 7 | SSE | 7 | 0 | 1 |
| 7/10/2008 | 12 | 74 | 5 | SE | 7 | 1 | 18.5 | 37 | 1 | SE | 2 | 0 | 0 |
| 8/10/2008 | 15 | 60 | 0 | ENE | 9 | 0 | 23 | 26 | 0 | N | 4 | 0 | 0 |
| 9/10/2008 | 15.5 | 63 | 1 | SW | 9 | 1 | 21.3 | 36 | 3 | SSE | 2 | 0 | 0 |
| 10/10/2008 | 14.7 | 65 | 0 | ENE | 4 | 0 | 23.5 | 31 | 3 | NW | 2 | 0 | 0 |
| 11/10/2008 | 13 | 73 | 6 | SW | 11 | 1 | 17.9 | 43 | 7 | SW | 11 | 1 | 3 |
| 12/10/2008 | 13.2 | 65 | 6 | SSW | 9 | 1 | 17 | 39 | 7 | SSW | 9 | 2 | 2 |
| 13/10/2008 | 13.4 | 56 | 1 | S | 4 | 0 | 19.1 | 35 | 4 | SSE | 9 | 1 | 1 |
| 14/10/2008 | 16.4 | 52 | 0 | SE | 15 | 2 | 25.4 | 27 | 0 | NE | 4 | 0 | 0 |
| 15/10/2008 | 21.6 | 35 | 5 | NNE | 19 | 0 | 32.9 | 11 | 5 | NNW | 37 | 0 | 0 |
| 16/10/2008 | 15.8 | 79 | 8 | SW | 7 | 1 | 20.6 | 53 | 6 | SSW | 7 | 2 | 2 |
| 17/10/2008 | 13.4 | 70 | 8 | SW | 4 | 0 | 22.8 | 33 | 0 | SSW | 4 | 1 | 1 |
| 18/10/2008 | 17.8 | 63 | 7 | SSW | 15 | 0 | 24.5 | 31 | 3 | S | 2 | 0 | 0 |
| 19/10/2008 | 18.6 | 61 | 0 | ESE | 4 | 0 | 29.5 | 26 | 4 | SSE | 4 | 0 | 0 |
| 20/10/2008 | 20.4 | 50 | 6 | NE | 33 | 0 | 29.4 | 25 | 7 | N | 15 | 0 | 0 |
| 21/10/2008 | 24.5 | 41 | 7 | NNE | 22 | 0 | 30 | 29 | 4 | N | 11 | 0 | 0 |
| 22/10/2008 | 23.3 | 49 | 4 | NNW | 4 | 0 | 28.9 | 31 | 8 | WNW | 4 | 0 | 0 |
| 23/10/2008 | 22.2 | 57 | 7 | E | 2 | 0 | 28.9 | 37 | 8 | N | 15 | 0 | 0 |
| 24/10/2008 | 17.5 | 85 | 8 | NW | 7 | 0 | 19 | 73 | 7 | NNW | 22 | 0 | 0 |
| 25/10/2008 | 15.2 | 62 | 5 | WSW | 15 | 0 | 19.7 | 40 | 5 | SW | 2 | 0 | 0 |
| 26/10/2008 | 12.5 | 72 | 7 |  | Calm | 0 | 22 | 37 | 0 | NW | 7 | 0 | 0 |
| 27/10/2008 | 13.4 | 79 | 1 | S | 4 | 0 | 23.9 | 32 | 3 | SE | 7 | 1 | 2 |
| 28/10/2008 | 18 | 65 | 5 | SE | 26 | 0 | 24.5 | 37 | 6 | SE | 15 | 0 | 0 |
| 29/10/2008 | 15.5 | 79 | 7 | SSW | 19 | 0 | 17.9 | 73 | 7 | SW | 15 | 0 | 0 |
| 30/10/2008 | 12 | 65 | 5 | SSE | 7 | 1 | 10.8 | 86 | 8 | W | 4 | 0 | 0 |
| 31/10/2008 | 12.4 | 74 | 7 | SE | 15 | 1 | 17 | 52 | 7 | SE | 2 | 0 | 0 |
| 1/11/2008 | 11.4 | 82 | 3 | ESE | 2 | 0 | 19.8 | 31 | 3 | SE | 2 | 0 | 0 |
| 2/11/2008 | 15.5 | 61 | 1 | NNE | 9 | 0 | 26.4 | 19 | 5 | NW | 2 | 0 | 0 |
| 3/11/2008 | 13.8 | 47 | 1 | SW | 7 | 1 | 17 | 42 | 7 | W | 9 | 0 | 0 |
| 4/11/2008 | 14.3 | 59 | 2 | WNW | 19 | 0 | 19.2 | 42 | 5 | WNW | 19 | 0 | 0 |
| 5/11/2008 | 14 | 73 | 5 | NW | 22 | 0 | 14.4 | 70 | 8 | WNW | 19 | 0 | 0 |
| 6/11/2008 | 10.8 | 83 | 8 | WSW | 4 | 0 | 17.5 | 46 | 6 | SW | 22 | 0 | 0 |
| 7/11/2008 | 12 | 61 | 3 | S | 9 | 0 | 16.3 | 48 | 8 | S | 7 | 1 | 1 |
| 8/11/2008 | 14.5 | 68 | 1 |  | Calm | 0 | 25.6 | 32 | 0 | NW | 4 | 0 | 0 |
| 9/11/2008 | 17.1 | 70 | 1 | SE | 2 | 0 | 29.6 | 22 | 5 | NW | 2 | 0 | 0 |
| 10/11/2008 | 18.4 | 67 | 1 | ESE | 2 | 0 | 28.2 | 31 | 8 | SSW | 2 | 0 | 0 |
| 11/11/2008 | 15.3 | 71 | 8 | ESE | 15 | 0 | 19 | 51 | 8 | SSW | 22 | 0 | 0 |
| 12/11/2008 | 12.6 | 71 | 8 | SSE | 7 | 0 | 20 | 33 | 5 | S | 4 | 0 | 0 |
| 13/11/2008 | 12.8 | 58 | 5 | SSE | 11 | 1 | 19 | 29 | 2 | SSE | 4 | 0 | 0 |
| 14/11/2008 | 13.4 | 57 | 5 | SE | 11 | 0 | 20.7 | 27 | 6 | SE | 11 | 0 | 0 |
| 15/11/2008 | 12.7 | 73 | 7 | SE | 15 | 0 | 20.1 | 38 | 6 | SSE | 15 | 0 | 0 |
| 16/11/2008 | 14.1 | 53 | 4 | ESE | 26 | 0 | 22.1 | 27 | 5 | SE | 15 | 1 | 0 |
| 17/11/2008 | 13.8 | 65 | 7 | SE | 19 | 0 | 21.7 | 35 | 2 | SE | 15 | 0 | 0 |
| 18/11/2008 | 14 | 68 | 6 | ESE | 7 | 0 | 23 | 29 | 1 | E | 2 | 0 | 0 |
| 19/11/2008 | 17.8 | 58 | 3 | ENE | 7 | 0 | 28.1 | 21 | 6 | NNW | 11 | 0 | 0 |
| 20/11/2008 | 18.9 | 66 | 1 | ESE | 15 | 0 | 25.9 | 37 | 5 | NW | 22 | 0 | 0 |
| 21/11/2008 | 16.9 | 67 | 2 | W | 4 | 0 | 22.5 | 36 | 7 | WSW | 2 | 0 | 0 |
| 22/11/2008 | 19.2 | 70 | 6 |  | Calm | 0 | 25.8 | 26 | 6 | NNW | 19 | 0 | 0 |
| 23/11/2008 | 15.1 | 86 | 7 |  | Calm | 0 | 22 | 42 | 5 | WNW | 19 | 0 | 0 |
| 24/11/2008 | 14 | 73 | 7 | SW | 11 | 2 | 20.2 | 51 | 7 | WNW | 9 | 0 | 0 |
| 25/11/2008 | 12.9 | 88 | 8 |  | Calm | 0 | 21.5 | 40 | 6 | W | 2 | 0 | 0 |
| 26/11/2008 | 15 | 80 | 7 | WSW | 7 | 1 | 17 | 67 | 6 | WSW | 2 | 0 | 1 |
| 27/11/2008 | 14.6 | 82 | 7 | SSW | 4 | 1 | 21 | 39 | 5 | SSW | 11 | 2 | 1 |
| 28/11/2008 | 14.4 | 72 | 5 |  | Calm | 0 | 22.8 | 30 | 1 | S | 9 | 1 | 1 |
| 29/11/2008 | 16.3 | 62 | 0 | SSW | 4 | 0 | 26 | 28 | 1 | SW | 15 | 2 | 3 |
| 30/11/2008 | 14.4 | 54 | 1 | SSE | 9 | 0 | 22.9 | 57 | 3 | SSW | 7 | 1 | 2 |
| 1/12/2008 | 13.8 | 61 | 6 | SE | 7 | 0 | 24 | 25 | 1 | NE | 4 | 0 | 0 |
| 2/12/2008 | 14.5 | 73 | 7 | SSE | 4 | 0 | 28.5 | 17 | 3 | ENE | 4 | 0 | 0 |
| 3/12/2008 | 20.2 | 73 | 5 | NNW | 4 | 0 | 24 | 33 | 3 | W | 19 | 0 | 0 |
| 4/12/2008 | 17.6 | 70 | 7 | NW | 15 | 0 | 22.9 | 40 | 6 | NW | 22 | 0 | 0 |
| 5/12/2008 | 14.8 | 59 | 6 | W | 4 | 0 | 22 | 30 | 5 | SSW | 2 | 0 | 0 |
| 6/12/2008 | 15.6 | 60 | 7 | ENE | 4 | 0 | 24.5 | 28 | 0 | E | 4 | 0 | 0 |
| 7/12/2008 | 19.9 | 49 | 0 | NNE | 11 | 0 | 31.1 | 19 | 5 | NNW | 19 | 0 | 0 |
| 8/12/2008 | 17.2 | 55 | 2 | S | 11 | 2 | 23 | 36 | 7 | SW | 7 | 1 | 1 |
| 9/12/2008 | 16.1 | 55 | 5 | SSW | 4 | 0 | 21.6 | 32 | 6 | NW | 15 | 0 | 0 |
| 10/12/2008 | 15.4 | 65 | 7 | SE | 11 | 1 | 19.2 | 42 | 7 | SE | 11 | 0 | 0 |
| 11/12/2008 | 14.6 | 83 | 8 | S | 11 | 2 | 23 | 31 | 5 | S | 4 | 0 | 1 |
| 12/12/2008 | 17.5 | 57 | 0 | ENE | 2 | 0 | 25 | 32 | 5 | SSW | 4 | 0 | 0 |
| 13/12/2008 | 20.7 | 56 | 0 | ENE | 2 | 0 | 30.7 | 27 | 3 | SSW | 7 | 1 | 1 |
| 14/12/2008 | 23.1 | 55 | 0 | ENE | 4 | 0 | 33.7 | 17 | 3 | NNW | 4 | 0 | 0 |
| 15/12/2008 | 17.8 | 77 | 7 | S | 11 | 1 | 27 | 42 | 3 | S | 15 | 1 | 0 |
| 16/12/2008 | 15.7 | 75 | 8 | SSE | 11 | 0 | 24.1 | 31 | 1 | S | 4 | 0 | 0 |
| 17/12/2008 | 15.7 | 58 | 1 | SSE | 4 | 0 | 25 | 23 | 0 | S | 4 | 0 | 0 |
| 18/12/2008 | 17 | 59 | 0 | ENE | 4 | 0 | 27.5 | 23 | 2 | ENE | 7 | 0 | 0 |
| 19/12/2008 | 24.5 | 30 | 0 | NE | 4 | 0 | 34.9 | 11 | 1 | NW | 11 | 0 | 0 |
| 20/12/2008 | 20.2 | 60 | 3 | NNW | 19 | 0 | 14.5 | 91 | 8 | WNW | 11 | 0 | 0 |
| 21/12/2008 | 13.6 | 71 | 8 | S | 11 | 1 | 20 | 45 | 4 | NW | 4 | 0 | 0 |
| 22/12/2008 | 16.8 | 56 | 1 | SE | 4 | 0 | 26.5 | 24 | 1 | NE | 7 | 0 | 0 |
| 23/12/2008 | 18.2 | 61 | 1 | SW | 2 | 0 | 29.5 | 21 | 6 | SW | 7 | 1 | 1 |
| 24/12/2008 | 18 | 61 | 4 | ENE | 4 | 0 | 28.2 | 23 | 3 | SW | 2 | 0 | 0 |
| 25/12/2008 | 19.8 | 54 | 0 | S | 11 | 1 | 29.5 | 19 | 0 | SW | 4 | 0 | 1 |
| 26/12/2008 | 17.3 | 63 | 1 | SSE | 4 | 0 | 27.4 | 19 | 0 | S | 4 | 0 | 1 |
| 27/12/2008 | 20 | 55 | 0 |  | Calm | 0 | 30.8 | 25 | 0 | S | 15 | 1 | 0 |
| 28/12/2008 | 17.5 | 73 | 8 | SE | 7 | 0 | 27.9 | 28 | 0 | E | 2 | 0 | 0 |
| 29/12/2008 | 19 | 51 | 0 | ESE | 4 | 0 | 31.6 | 16 | 0 | ESE | 4 | 0 | 0 |
| 30/12/2008 | 20 | 46 | 0 | SE | 7 | 1 | 34.9 | 16 | 0 | N | 11 | 0 | 0 |
| 31/12/2008 | 20.8 | 59 | 0 | SSE | 11 | 0 | 30 | 30 | 3 | SE | 11 | 1 | 1 |
| 1/01/2009 | 20 | 50 | 0 | E | 19 | 0 | 32 | 24 | 0 | ESE | 15 | 0 | 0 |
| 2/01/2009 | 22.7 | 53 | 0 | SE | 2 | 0 | 37.2 | 17 | 1 | S | 4 | 0 | 0 |
| 3/01/2009 | 19.3 | 70 | 6 | SSE | 19 | 0 | 30 | 25 | 0 | S | 19 | 0 | 0 |
| 4/01/2009 | 19 | 59 | 0 | ESE | 9 | 0 | 30.7 | 24 | 0 | SE | 7 | 1 | 1 |
| 5/01/2009 | 24.1 | 33 | 0 | E | 2 | 0 | 34.6 | 16 | 0 |  | Calm | 0 | 0 |
| 6/01/2009 | 21 | 61 | 0 | ESE | 2 | 0 | 31.7 | 24 | 0 | SE | 7 | 1 | 0 |
| 7/01/2009 | 19.4 | 61 | 5 | SE | 19 | 0 | 29.9 | 28 | 1 | SE | 11 | 0 | 0 |
| 8/01/2009 | 19.9 | 65 | 0 | SE | 15 | 1 | 32.5 | 28 | 0 | SE | 15 | 1 | 0 |
| 9/01/2009 | 20.6 | 62 | 1 | SE | 4 | 0 | 33.8 | 20 | 1 | SSW | 11 | 1 | 1 |
| 10/01/2009 | 20.5 | 64 | 0 | SSE | 2 | 0 | 33.5 | 16 | 0 | S | 4 | 0 | 0 |
| 11/01/2009 | 22.5 | 49 | 0 | ENE | 2 | 0 | 36.5 | 11 | 0 | N | 2 | 0 | 0 |
| 12/01/2009 | 24.5 | 31 | 0 | SSW | 11 | 2 | 30.6 | 30 | 2 | WSW | 15 | 1 | 0 |
| 13/01/2009 | 16.6 | 65 | 5 | SSW | 4 | 0 | 25 | 27 | 0 | SSE | 4 | 0 | 0 |
| 14/01/2009 | 18 | 58 | 3 | SE | 11 | 1 | 26.5 | 31 | 4 | SE | 7 | 0 | 0 |
| 15/01/2009 | 19.1 | 59 | 0 | E | 11 | 0 | 32.5 | 21 | 0 | E | 9 | 0 | 0 |
| 16/01/2009 | 29 | 17 | 0 | NE | 26 | 0 | 40.7 | 9 | 0 | NNW | 22 | 0 | 0 |
| 17/01/2009 | 20.3 | 69 | 1 | SE | 19 | 0 | 32 | 32 | 4 | SW | 4 | 0 | 1 |
| 18/01/2009 | 20.4 | 68 | 6 | S | 15 | 1 | 26.7 | 42 | 7 | S | 22 | 0 | 0 |
| 19/01/2009 | 15.2 | 81 | 7 | SSE | 22 | 0 | 23 | 46 | 1 | SSE | 19 | 0 | 0 |
| 20/01/2009 | 15.2 | 67 | 1 | SE | 2 | 0 | 27.9 | 17 | 1 | SSW | 4 | 0 | 0 |
| 21/01/2009 | 18.1 | 70 | 1 | SW | 2 | 0 | 30.1 | 25 | 4 | WSW | 15 | 1 | 0 |
| 22/01/2009 | 19.6 | 71 | 6 | SW | 2 | 0 | 29 | 49 | 1 | SW | 7 | 3 | 2 |
| 23/01/2009 | 20.5 | 67 | 2 | SW | 7 | 1 | 29.6 | 30 | 0 | SSW | 11 | 1 | 1 |
| 24/01/2009 | 20 | 61 | 0 | E | 2 | 0 | 32.9 | 21 | 0 | NNW | 2 | 0 | 0 |
| 25/01/2009 | 19.4 | 63 | 0 | SE | 7 | 0 | 35.5 | 19 | 0 | SE | 7 | 0 | 0 |
| 26/01/2009 | 22.5 | 62 | 0 | SSW | 2 | 0 | 37.5 | 15 | 0 | WNW | 2 | 0 | 0 |
| 27/01/2009 | 19.2 | 62 | 2 | SE | 11 | 0 | 27.5 | 44 | 7 | SSE | 9 | 1 | 0 |
| 28/01/2009 | 19 | 84 | 8 | SE | 2 | 0 | 24 | 66 | 8 | ESE | 4 | 0 | 0 |
| 29/01/2009 | 16.2 | 94 | 8 | SE | 15 | 0 | 19.2 | 81 | 8 | SSE | 4 | 0 | 0 |
| 30/01/2009 | 17.4 | 81 | 8 | SE | 7 | 0 | 27 | 50 | 7 | SSE | 11 | 0 | 1 |
| 31/01/2009 | 20.2 | 84 | 8 | ESE | 4 | 0 | 28.9 | 53 | 6 | NNE | 7 | 0 | 0 |
| 1/02/2009 | 24 | 78 | 0 | ESE | 2 | 0 | 34 | 39 | 6 |  | Calm | 0 | 0 |
| 2/02/2009 | 27.6 | 55 | 0 | NW | 2 | 0 | 38.2 | 17 | 6 | NNW | 9 | 0 | 0 |
| 3/02/2009 | 25.4 | 62 | 0 | S | 2 | 0 | 35.5 | 24 | 0 | SSW | 2 | 0 | 0 |
| 4/02/2009 | 19.6 | 65 | 3 | SSE | 15 | 0 | 28.6 | 35 | 1 | SSE | 19 | 0 | 1 |
| 5/02/2009 | 17.3 | 58 | 0 | SE | 11 | 0 | 28.7 | 21 | 0 | ESE | 2 | 0 | 0 |
| 6/02/2009 | 17.2 | 63 | 8 | S | 19 | 0 | 26.2 | 27 | 1 | SW | 11 | 2 | 3 |
| 7/02/2009 | 16.5 | 61 | 5 | SSW | 11 | 2 | 24 | 27 | 0 | SSE | 4 | 0 | 0 |
| 8/02/2009 | 15.9 | 54 | 0 | SE | 19 | 0 | 27.8 | 20 | 0 | SE | 11 | 0 | 1 |
| 9/02/2009 | 17 | 61 | 0 | E | 15 | 0 |  |  |  |  |  |  |  |
| 10/02/2009 | 18.8 | 55 | 0 | ESE | 4 | 0 | 29.6 | 23 | 4 | ESE | 7 | 0 | 0 |
| 11/02/2009 | 23.8 | 31 | 1 | NE | 22 | 0 | 33.6 | 15 | 1 | SE | 2 | 0 | 0 |
| 12/02/2009 | 25.7 | 24 | 3 | N | 22 | 0 | 37.4 | 12 | 4 | NW | 7 | 0 | 0 |
| 13/02/2009 | 17 | 87 | 8 | S | 15 | 2 | 18 | 81 | 8 | SE | 19 | 0 | 0 |
| 14/02/2009 | 18 | 83 | 8 | SSE | 9 | 0 | 22.5 | 68 | 7 | SSE | 15 | 0 | 0 |
| 15/02/2009 | 24.8 | 50 | 0 | N | 7 | 0 | 36 | 19 | 1 | NNW | 11 | 0 | 0 |
| 16/02/2009 | 20.4 | 62 | 3 | NW | 2 | 0 | 29.5 | 31 | 4 | W | 4 | 0 | 0 |
| 17/02/2009 | 16.8 | 72 | 6 | SSE | 9 | 0 | 26.5 | 33 | 1 | SSE | 9 | 1 | 1 |
| 18/02/2009 | 16.4 | 69 | 3 | ESE | 11 | 0 | 25.7 | 26 | 0 | SE | 15 | 0 | 0 |
| 19/02/2009 | 16.2 | 64 | 0 | ESE | 15 | 0 | 27 | 24 | 0 | SSE | 4 | 0 | 0 |
| 20/02/2009 | 17.3 | 68 | 0 | ESE | 4 | 0 | 30.4 | 25 | 1 | ENE | 2 | 0 | 0 |
| 21/02/2009 | 22 | 45 | 0 | N | 4 | 0 |  |  |  |  |  |  |  |
| 22/02/2009 | 18 | 71 | 6 | S | 15 | 1 | 28.2 | 28 | 0 | S | 15 | 2 | 1 |
| 23/02/2009 | 15.6 | 63 | 6 | E | 4 | 0 | 28.6 | 31 | 4 | ENE | 4 | 0 | 0 |
| 24/02/2009 | 21.6 | 57 | 8 | SSE | 2 | 0 | 26.3 | 38 | 8 | WNW | 15 | 0 | 0 |
| 25/02/2009 | 16.9 | 68 | 7 | NNW | 11 | 0 | 18.5 | 67 | 8 | WSW | 15 | 1 | 0 |
| 26/02/2009 | 13.9 | 64 | 0 | SW | 2 | 0 | 21.8 | 35 | 0 | WSW | 7 | 1 | 1 |
| 27/02/2009 | 14.7 | 69 | 2 | SE | 7 | 0 | 26.5 | 27 | 1 | ENE | 2 | 0 | 0 |
| 28/02/2009 | 17.9 | 68 | 3 | SE | 11 | 0 | 32.4 | 23 | 5 | SSE | 11 | 1 | 1 |
| 1/03/2009 | 21.3 | 79 | 7 |  | Calm | 0 |  |  |  |  |  |  |  |
| 2/03/2009 | 15.2 | 82 | 7 | SSW | 9 | 2 | 19.4 | 52 | 6 | S | 15 | 2 | 0 |
| 3/03/2009 | 11.6 | 64 | 0 | SSE | 9 | 0 | 19 | 35 | 0 | SE | 4 | 0 | 0 |
| 4/03/2009 | 13.2 | 73 | 0 | E | 2 | 0 | 24.5 | 29 | 1 | ENE | 4 | 0 | 0 |
| 5/03/2009 | 15.6 | 71 | 0 | ENE | 2 | 0 |  |  |  |  |  |  |  |
| 6/03/2009 | 17.5 | 70 | 0 |  | Calm | 0 | 28.7 | 25 | 1 | S | 4 | 0 | 0 |
| 7/03/2009 | 15.4 | 75 | 7 | E | 9 | 0 | 24.8 | 26 | 0 | E | 11 | 0 | 0 |
| 8/03/2009 | 17.5 | 61 | 5 | NE | 7 | 0 | 29 | 23 | 5 | NW | 2 | 0 | 0 |
| 9/03/2009 | 20.5 | 42 | 0 | N | 2 | 0 | 32.4 | 17 | 4 | NNW | 2 | 0 | 0 |
| 10/03/2009 | 23.8 | 26 | 0 | NNE | 2 | 0 | 34.2 | 13 | 1 | NNE | 2 | 0 | 0 |
| 11/03/2009 | 22.9 | 32 | 4 | NW | 2 | 0 | 28.7 | 29 | 7 | SW | 2 | 0 | 0 |
| 12/03/2009 | 15.3 | 89 | 8 | SSE | 2 | 0 | 20.8 | 62 | 7 | SE | 2 | 0 | 0 |
| 13/03/2009 | 15.4 | 65 | 1 | SSE | 2 | 0 | 22.5 | 38 | 1 | SE | 4 | 0 | 0 |
| 14/03/2009 | 13.1 | 66 | 0 | ESE | 11 | 1 | 24.5 | 27 | 0 | ENE | 2 | 0 | 0 |
| 15/03/2009 | 17 | 58 | 5 | ESE | 4 | 0 | 31.5 | 21 | 5 | ENE | 7 | 0 | 0 |
| 16/03/2009 | 24.3 | 33 | 1 | NE | 11 | 0 | 35.8 | 16 | 1 | NW | 28 | 0 | 0 |
| 17/03/2009 | 18.7 | 70 | 5 | NW | 15 | 0 | 25.2 | 46 | 6 | NW | 28 | 0 | 0 |
| 18/03/2009 | 15.2 | 75 | 2 | SSW | 11 | 2 | 22.6 | 35 | 0 | SW | 9 | 2 | 2 |
| 19/03/2009 | 13.4 | 74 | 5 |  | Calm | 0 | 25.1 | 32 | 1 | WNW | 11 | 0 | 0 |
| 20/03/2009 | 16.7 | 68 | 1 | NW | 7 | 0 | 24.4 | 40 | 6 | W | 15 | 0 | 0 |
| 21/03/2009 | 12.1 | 92 | 5 |  | Calm | 0 | 21 | 35 | 0 | NW | 4 | 0 | 0 |
| 22/03/2009 | 13 | 75 | 0 |  | Calm | 0 | 24 | 31 | 0 | NW | 2 | 0 | 0 |
| 23/03/2009 | 15.5 | 74 | 1 | W | 2 | 0 | 24.6 | 42 | 5 | WSW | 4 | 1 | 1 |
| 24/03/2009 | 16 | 70 | 4 | SW | 2 | 0 | 24.6 | 32 | 2 | NW | 11 | 0 | 0 |
| 25/03/2009 | 15.8 | 88 | 8 | SSE | 2 | 0 | 22.8 | 40 | 6 | SW | 2 | 0 | 0 |
| 26/03/2009 | 15.9 | 75 | 2 | SE | 2 | 0 | 28.5 | 32 | 2 | NE | 2 | 0 | 0 |
| 27/03/2009 | 23.4 | 38 | 0 | NNE | 9 | 0 | 35.7 | 14 | 0 | NW | 19 | 0 | 0 |
| 28/03/2009 | 17 | 76 | 5 | S | 19 | 0 | 27 | 32 | 1 | S | 15 | 1 | 0 |
| 29/03/2009 | 15.3 | 67 | 8 | ESE | 19 | 0 | 20.5 | 43 | 7 | SE | 15 | 0 | 0 |
| 30/03/2009 | 15 | 91 | 8 | SSE | 9 | 1 | 21.9 | 54 | 7 | SSE | 4 | 0 | 0 |
| 31/03/2009 | 15 | 94 | 8 | SSE | 9 | 1 | 19.5 | 56 | 5 | SSW | 9 | 2 | 1 |
| 1/04/2009 | 13.5 | 57 | 1 | SSE | 15 | 1 | 19.5 | 32 | 0 | SSE | 2 | 0 | 0 |
| 2/04/2009 | 13.3 | 60 | 2 | SE | 4 | 0 | 21 | 29 | 1 | SE | 2 | 0 | 0 |
| 3/04/2009 | 15.1 | 64 | 0 | SE | 9 | 1 | 23.2 | 28 | 0 | SE | 9 | 0 | 1 |
| 4/04/2009 | 14.9 | 65 | 2 | ESE | 2 | 0 | 24 | 27 | 4 | SE | 7 | 0 | 0 |
| 5/04/2009 | 15 | 68 | 0 | E | 15 | 0 | 23 | 29 | 0 | SE | 7 | 0 | 0 |
| 6/04/2009 | 14.8 | 70 | 1 | E | 2 | 0 | 24.9 | 25 | 1 | E | 2 | 0 | 0 |
| 7/04/2009 | 18.6 | 55 | 0 | NNE | 15 | 0 | 27.5 | 20 | 1 | NNE | 2 | 0 | 0 |
| 8/04/2009 | 20 | 33 | 6 | N | 11 | 0 | 29.3 | 16 | 5 | NNW | 4 | 0 | 0 |
| 9/04/2009 | 17.5 | 68 | 3 | SE | 4 | 0 | 27 | 30 | 0 |  | Calm | 0 | 0 |
| 10/04/2009 | 16.7 | 68 | 4 | ENE | 2 | 0 | 28.9 | 24 | 1 | NW | 4 | 0 | 0 |
| 11/04/2009 | 24 | 19 | 7 | NNE | 11 | 0 | 31.8 | 8 | 7 | N | 11 | 0 | 0 |
| 12/04/2009 | 23 | 32 | 0 | ENE | 19 | 0 | 30.2 | 15 | 1 | NE | 11 | 0 | 0 |
| 13/04/2009 | 18.1 | 72 | 3 | ESE | 7 | 0 | 27 | 28 | 7 | ESE | 2 | 0 | 0 |
| 14/04/2009 | 16.8 | 85 | 8 | SE | 9 | 0 | 23.4 | 50 | 3 | SE | 4 | 0 | 0 |
| 15/04/2009 | 12.8 | 68 | 1 | ESE | 2 | 0 | 21.5 | 33 | 0 | S | 7 | 2 | 0 |
| 16/04/2009 | 13 | 73 | 0 | SE | 2 | 0 | 23.5 | 42 | 1 | SSE | 2 | 0 | 0 |
| 17/04/2009 | 18 | 65 | 1 | ENE | 2 | 0 | 27 | 35 | 6 |  | Calm | 0 | 0 |
| 18/04/2009 | 21 | 58 | 5 | NE | 9 | 0 | 22.7 | 72 | 7 |  | Calm | 0 | 0 |
| 19/04/2009 | 20.2 | 66 | 0 |  | Calm | 0 | 27.6 | 30 | 1 | SE | 2 | 0 | 0 |
| 20/04/2009 | 16.6 | 69 | 0 |  | Calm | 0 | 27.2 | 31 | 6 | SE | 2 | 0 | 0 |
| 21/04/2009 | 21 | 50 | 2 | NNE | 11 | 0 | 27.5 | 31 | 6 | NNE | 4 | 0 | 0 |
| 22/04/2009 | 16.4 | 71 | 7 | SW | 9 | 2 | 17.5 | 42 | 5 | SW | 19 | 3 | 2 |
| 23/04/2009 | 11.3 | 62 | 7 | SW | 4 | 1 | 17.5 | 50 | 7 | SSW | 4 | 2 | 2 |
| 24/04/2009 | 14.1 | 83 | 8 | SSW | 2 | 1 | 23.5 | 42 | 0 | WNW | 4 | 1 | 3 |
| 25/04/2009 | 15 | 79 | 7 | SW | 4 | 4 | 20.9 | 50 | 4 | S | 7 | 1 | 0 |
| 26/04/2009 | 15 | 79 | 7 |  | Calm | 0 | 23 | 42 | 1 |  | Calm | 0 | 0 |
| 27/04/2009 | 15.6 | 73 | 0 |  | Calm | 0 | 25.6 | 30 | 4 | NW | 4 | 0 | 0 |
| 28/04/2009 | 14.5 | 77 | 1 | SE | 4 | 0 | 24.5 | 25 | 0 |  | Calm | 0 | 0 |
| 29/04/2009 | 18 | 51 | 7 |  | Calm | 0 | 25.6 | 27 | 6 | NW | 11 | 0 | 0 |
| 30/04/2009 | 15 | 69 | 5 | SE | 7 | 0 | 19.6 | 37 | 0 | SSE | 4 | 0 | 0 |

## Interpretation

Interpretation and analysis of the data is (at this stage) being undertaken by the Department of Environment and Conservation.  Unfortunately access to statisticians in the Department of Agriculture and Food is limited until September, after the analysis of the 2008 crop and livestock trial data.

## Discussion

Wait for interpretation.

## Extrapolation

It is intended that this data be considered by the modelling team for possibly a predictive tool.  If this is the case it will be managed under Project 32-2009-01.

## Learnings

Would the Department do the same process again?  This is a difficult question to answer.  Logistically the resources required are substantial and there is always likely to be debate over the validity of the results.  The Departments limited experience with odour is clearly an area for strengthening.  In the absence of technical equipment that can perform this function this may be the best way to collect this information.

Note:  The Department of Primary Industry and Fisheries in QLD are trying to develop an electronic nose.  A watching brief should be maintained on this project to see when/if it is likely to generate a product.

Industry Development Report 32-2006-01

Odour Monitoring

Narrogin Town Site

Indicative Results

By Pat Page and Claire Coffey

July 2009

Disclaimer:

While all reasonable care has been taken in the preparation of the material in this document, the Western Australian Government and its officers accept no responsibility for any errors or omissions it may contain, whether caused by negligence, or otherwise or for any loss, however caused, sustained by any person who relies on it.

© State of Western Australia, 2009